# CCNA Practice Questions: Set 1 Answer Key

Do not open until you have answered. Items marked CHECK may be a flawed question or a garbled transcript. Verify them against Jeremy's videos before trusting the answer.

| Q | Ans | Why | CHECK |
| --- | --- | --- | --- |
| 1 | B | Restrict drops the traffic, counts violations and logs/sends SNMP. Shutdown = err-disabled. Protect drops silently. |  |
| 2 | C | Video answer: Listening (takes part in topology decisions, no MAC learning). Learning starts filling the MAC table. | Blocking also receives BPDUs and does not learn MACs, so this question is ambiguous. Check how Jeremy words it. |
| 3 | B | Longest prefix wins. /20 covers 172.16.32.0 to 172.16.47.255, so it matches. /24 covers only 172.16.32.x. |  |
| 4 | C | Hello packets handle neighbour discovery and DR/BDR election. |  |
| 5 | B | switchport nonegotiate stops DTP frames. |  |
| 6 | B | Untagged native VLAN traffic can be put in the wrong VLAN, causing VLAN leakage. |  |
| 7 | B | FE80::/10 is link-local. |  |
| 8 | B | OSPF = 110. |  |
| 9 | B | PAgP is Cisco proprietary. LACP is the open standard (802.3ad). |  |
| 10 | C | PAT (NAT overload) translates many inside hosts using Layer 4 ports. |  |
| 11 | D | All ACLs are processed in order and stop at the first match. |  |
| 12 | C | Autonomous APs manage themselves. Lightweight APs use a controller. |  |
| 13 | C | IPv6 uses NDP instead of ARP. | Option B was garbled in the transcript. Shown as RP. |
| 14 | B | If a PortFast port receives a BPDU, BPDU Guard err-disables it. |  |
| 15 | B | show ip ssh shows the SSH version and parameters. |  |
| 16 | A | /26 blocks of 64: 0-63, 64-127, 128-191, 192-255. .190 is in 128-191. | Transcript read 192.168.10.19, which would be 192.168.10.0/26 (D). The video's explanation says 190, so I used .190. |
| 17 | D | Exchange = DBD summaries swapped, not fully synchronised yet. | Strictly, DBD summaries are exchanged in Exchange. After that, Loading, then Full. |
| 18 | D | Cisco shortcut: spanning-tree vlan 1 root primary. |  |
| 19 | A | The CAM table updates to the newest port. Repeated moves indicate flapping. |  |
| 20 | B | SNMP uses UDP 161. Traps use UDP 162. |  |
| 21 | C | The Active router forwards traffic. |  |
| 22 | D | The Transport layer. TCP provides reliability. |  |
| 23 | C | show cdp neighbors lists adjacent Cisco devices. |  |
| 24 | B | Trusted ports may receive DHCP offers/ACKs, typically uplinks to the real server. |  |
| 25 | C | Higher AD than the primary route, so they install only if the primary fails. |  |
| 26 | B | Video answer: a superior BPDU triggers STP recalculation and roles may change. CAM flushing can come later. | Weak question. A superior BPDU arriving on the root port normally just updates the root information. Check before trusting this one. |
| 27 | C | AD is compared before metric. Static AD 1 wins. |  |
| 28 | B | Native VLAN traffic is sent untagged even if it is also in the allowed list. |  |
| 29 | C | Passive waits for the other side to start. Passive + passive = no negotiation. |  |
| 30 | B | Broadcast networks (Ethernet) elect a DR/BDR. |  |
| 31 | B | TTL reaches 0, the router drops the packet and sends ICMP Time Exceeded. |  |
| 32 | B | Sticky learns dynamically and writes the secure MACs into the running config. |  |
| 33 | C | FE80::/10 is never routed. |  |
| 34 | B | Lower priority wins. 28672 beats 32768. |  |
| 35 | A | Classic: ExStart stuck = MTU mismatch. |  |
| 36 | A | A single host uses wildcard 0.0.0.0. | Addresses in the transcript were garbled. I rebuilt them as 10.1.1.30 variants. |
| 37 | A | CDP can detect and warn about a native VLAN mismatch. |  |
| 38 | A | show standby brief. |  |
| 39 | B | Unknown unicast flooding. |  |
| 40 | C | Static mappings stay until removed. |  |
| 41 | C | The high AD makes it a backup route. | The transcript garbled the next hop. I used 10.1.1.2. |
| 42 | B | 224.0.0.5 = OSPF all SPF routers. |  |
| 43 | A | Classic neighbour requirements. | Not the full list (subnet, authentication, stub flags and MTU also matter), but A is the best answer here. |
| 44 | C | DORA: Discover, Offer, Request, ACK. |  |
| 45 | B | show mac address-table. |  |
| 46 | C | The longest matching prefix wins. | Addresses were garbled in the transcript. I rebuilt them from the answer. |
| 47 | B | Dynamic auto waits for the other side to negotiate. |  |
| 48 | A | DHCP snooping blocks rogue DHCP offers. |  |
| 49 | B | The designated port is the one with the lowest root path cost. |  |
| 50 | A | A directed broadcast targets a specific subnet's broadcast address. A limited broadcast is 255.255.255.255. | The /24 mask was not in the transcript. I added it. |
| 51 | C | shutdown disables the SVI. |  |
| 52 | B | Active = LACP. On = static. A mismatch stops the bundle forming. | A and B are close. B is the cleaner answer. |
| 53 | B | You receive their Hellos, but your own router ID is not in their Hello. |  |
| 54 | A | Without a default route, remote traffic is dropped. | Hosts normally use their own default gateway. Treat this as a router-routing question. |
| 55 | C | A CAM overflow attack causes flooding and CPU issues. |  |
| 56 | B | An MTU mismatch often prevents full adjacency. |  |
| 57 | B | APIPA means DHCP failed. |  |
| 58 | B | A standard ACL is processed top-down. | The host address was garbled in the transcript. I used 10.1.1.5. |
| 59 | A | A Layer 2 loop causes duplicate frames and broadcast storms. |  |
| 60 | C | The AD 1 static was already preferred over OSPF and still is. | Poor question wording. The static route never needed OSPF to fail. |
| 61 | B | If VLAN 10 is native, the sub-interface must be marked native. |  |
| 62 | B | BPDU Guard shuts the port down. |  |
| 63 | B | Name resolution (DNS). | Option D was garbled. Shown as DNAT. |
| 64 | A | Video answer: passive interface suppresses advertisements. | Questionable. A passive interface sends no Hellos, so no adjacency would form at all (see Q96). Do not rely on this one. |
| 65 | A | Classic allowed-VLAN omission. |  |
| 66 | B | Protects against rogue switches. |  |
| 67 | B | Video answer: traceroute depends on TTL expiry. | Questionable. If a traceroute stops at hop 4, that device usually does not reply (filtered ICMP, or no onward route). Check before trusting this. |
| 68 | A | A huge exam favourite. |  |
| 69 | A | Bad NAT often breaks inside traffic. |  |
| 70 | A | A hidden EtherChannel killer. | This is the video's view. Check it against your notes. |
| 71 | B | DROthers stay 2-Way with each other. | The options were garbled. I rebuilt them from the answer. |
| 72 | B | Huge Cisco favourite. |  |
| 73 | B | A native mismatch can leak traffic. |  |
| 74 | B | Direction matters. Classic exam trap. |  |
| 75 | A | Default STP convergence is slow unless you use RSTP. |  |
| 76 | B | Video answer: nonegotiate suppresses DTP even with dynamic desirable. | Likely wrong. On real Catalyst IOS, `nonegotiate` is rejected unless the mode is trunk or access (conflict with dynamic modes). Check before trusting this. |
| 77 | C | Same address on multiple devices. Routing delivers to the nearest. |  |
| 78 | C | Equal-cost OSPF routes can both be installed (ECMP). |  |
| 79 | A | Forcing access mode prevents trunk formation. Nonegotiate only stops DTP frames. |  |
| 80 | B | Same prefix length, so AD decides. OSPF 110 beats static 200. |  |
| 81 | C | PortFast on a switch-to-switch link can create loops. |  |
| 82 | C | OSPF cost, commonly derived from bandwidth. |  |
| 83 | A | /21 = 255.255.248.0. The inverse is 0.0.7.255. | Options were garbled. Rebuilt as above. |
| 84 | B | Administratively down means manually shut. |  |
| 85 | B | The active router owns the virtual MAC/IP response. |  |
| 86 | A | Layer 2 switches use it for management reachability. |  |
| 87 | A | Classic mitigation. |  |
| 88 | B | Frames for the switch itself are punted to the CPU/control plane. |  |
| 89 | B | RSTP improves convergence significantly. |  |
| 90 | A | The default gateway is delivered by DHCP option 3. |  |
| 91 | B | show spanning-tree shows root information. | `show spanning-tree root` is also a valid command on many Catalyst IOS versions. B is the video's answer. |
| 92 | B | There is an implicit deny at the end of every ACL. |  |
| 93 | C | Full = LSDBs synchronised. |  |
| 94 | B | MAC flapping often points to loops or instability. |  |
| 95 | C | It ARPs for the default gateway's IP to learn its MAC. | Strictly it ARPs for the gateway's IP and learns its MAC. C is the intended answer. |
| 96 | A | No Hellos, so no adjacency forms. The subnet is still advertised. |  |
| 97 | A | show port-security address. |  |
| 98 | B | OSPF uses IP protocol 89. (EIGRP uses 88.) |  |
| 99 | B | The allowed VLAN list still controls forwarding. |  |
| 100 | A | Late collisions plus slowness. |  |