# CCNA Practice Questions: Set 1 (Q1 to Q100)

Answers are in the separate file CCNA_Set1_Answer_Key.docx. Mark your answer, then check.

Q1. A network engineer configures an interface with switchport port-security violation restrict. What happens when a violating MAC address is detected?

A.  The port goes err-disabled.

B.  The violating frames are dropped and a log/SNMP trap is generated.

C.  The MAC address is added dynamically to the secure table.

D.  The switch shuts down VLAN access.

Q2. Which STP port state receives BPDUs but does not learn MAC addresses?

A.  Forwarding

B.  Blocking

C.  Listening

D.  Learning

Q3. A router receives a packet for 172.16.33.10 and has these routes: 172.16.0.0/16, 172.16.32.0/20, 172.16.32.0/24. Which route is selected?

A.  172.16.0.0/16

B.  172.16.32.0/20

C.  172.16.32.0/24

D.  Equal-cost load balancing occurs

Q4. Which OSPF packet type is used to elect the DR/BDR?

A.  LSU

B.  DBD

C.  Hello

D.  LSR

Q5. Which command prevents DTP negotiation on a switchport?

A.  switchport mode access

B.  switchport nonegotiate

C.  no dtp enable

D.  spanning-tree portfast

Q6. What occurs if native VLANs mismatch on an 802.1Q trunk?

A.  The trunk fails to form.

B.  Traffic may leak into the wrong VLAN.

C.  STP disables the trunk automatically.

D.  DTP renegotiates the native VLAN.

Q7. Which IPv6 address is link-local?

A.  2001:DB8::1

B.  FE80::1

C.  FF02::1

D.  FC00::1

Q8. What is the default administrative distance of OSPF?

A.  90

B.  110

C.  120

D.  115

Q9. Which EtherChannel protocol is Cisco proprietary?

A.  LACP

B.  PAgP

C.  802.1X

D.  UDLD

Q10. Which feature allows multiple hosts to share one public IP address using ports?

A.  Static NAT

B.  Dynamic NAT

C.  PAT

D.  Policy NAT

Q11. Which ACL is processed top-down and stops at the first match?

A.  Standard only

B.  Extended only

C.  Numbered only

D.  All Cisco ACLs

Q12. Which wireless architecture places control logic on the access points instead of a central controller?

A.  Split-MAC

B.  Lightweight

C.  Autonomous

D.  CAPWAP

Q13. Which protocol maps IPv6 addresses to MAC addresses?

A.  ARP

B.  RP

C.  NDP

D.  ICMP redirect

Q14. What is the purpose of BPDU Guard?

A.  Prevent loops on the root bridge

B.  Disable PortFast ports that receive BPDUs

C.  Change root bridge priority

D.  Filter VLAN traffic

Q15. Which command verifies the SSH version configured on Cisco IOS?

A.  show ssh

B.  show ip ssh

C.  show crypto ssh

D.  show run ssh

Q16. Which subnet includes 192.168.10.190?

A.  192.168.10.128/26

B.  192.168.10.64/26

C.  192.168.10.192/26

D.  192.168.10.0/26

Q17. Which OSPF state means routers have exchanged LSDB summaries but are not yet fully adjacent?

A.  Init

B.  2-Way

C.  ExStart

D.  Exchange

Q18. Which command makes a switch the STP root primary for VLAN 1?

A.  spanning-tree root primary

B.  spanning-tree vlan 1 priority 0

C.  spanning-tree root bridge primary

D.  spanning-tree vlan 1 root primary

Q19. A switch learns a MAC on Fa0/5 and later sees the same MAC on Fa0/8. What happens?

A.  MAC flapping occurs and the CAM table updates.

B.  Port security disables the switch.

C.  ARP clears the table.

D.  The switch reboots STP.

Q20. Which transport protocol and port does SNMP use for polling?

A.  TCP 161

B.  UDP 161

C.  TCP 443

D.  UDP 22

Q21. Which HSRP state forwards virtual gateway traffic?

A.  Listen

B.  Speak

C.  Active

D.  Standby

Q22. Which layer of the TCP/IP model handles end-to-end reliability?

A.  Internet

B.  Network access

C.  Application

D.  Transport

Q23. Which command verifies CDP neighbours?

A.  show cdp

B.  show cdp interface

C.  show cdp neighbors

D.  show discovery neighbors

Q24. What does ip dhcp snooping trust do?

A.  Blocks DHCP offers

B.  Allows DHCP server replies on the interface

C.  Disables DHCP relay

D.  Encrypts DHCP traffic

Q25. Which statement about floating static routes is true?

A.  They have a lower AD than dynamic routes.

B.  They are always preferred over OSPF.

C.  They use a higher AD so they act as backup routes.

D.  They require BGP to function.

Q26. A switch receives superior BPDUs on its root port. What occurs first?

A.  The root port immediately goes to forwarding.

B.  Root bridge election restarts and port roles may be recalculated.

C.  The CAM table is flushed before recalculation.

D.  EtherChannel renegotiation begins.

Q27. Which route is preferred? OSPF route (AD 110, metric 5), EIGRP route (AD 90, metric 5000), static route (AD 1).

A.  OSPF

B.  EIGRP

C.  Static

D.  Lowest metric wins

Q28. A trunk is configured with switchport trunk allowed vlan 10,20 and switchport trunk native vlan 20. What happens to VLAN 20 traffic?

A.  Tagged only

B.  Untagged, because it is the native VLAN

C.  Blocked due to overlap

D.  Converted to VLAN 1

Q29. What happens if both ends of an EtherChannel use LACP passive?

A.  It forms successfully.

B.  It forms only if trunking is enabled.

C.  It does not form.

D.  It falls back to PAgP.

Q30. Which OSPF network type elects a DR/BDR by default?

A.  Point-to-point

B.  Broadcast

C.  Point-to-multipoint

D.  Loopback

Q31. A router receives a packet with TTL = 1. What happens after the forwarding decision?

A.  It is forwarded normally.

B.  TTL is decremented to 0, the packet is dropped and a Time Exceeded message is sent.

C.  TTL is reset to the default.

D.  An ARP request is sent.

Q32. Which statement about port security sticky MAC addresses is true?

A.  Only static MACs are allowed.

B.  It learns MACs dynamically and adds them to the running config.

C.  It disables violations.

D.  It works only on trunk ports.

Q33. Which IPv6 address type can never be routed beyond the local segment?

A.  Global unicast

B.  Unique local

C.  Link-local

D.  Anycast

Q34. A switch has the default root bridge priority of 32768. Another switch is configured with spanning-tree vlan 1 priority 28672. Which becomes root?

A.  The original switch

B.  The new switch

C.  Lower MAC decides first

D.  The election fails

Q35. What commonly causes OSPF neighbours to stay stuck in ExStart?

A.  MTU mismatch

B.  Missing default route

C.  ACL deny ip any any

D.  VLAN pruning

Q36. Which ACL entry matches only host 10.1.1.30 using a wildcard mask?

A.  10.1.1.30 0.0.0.0

B.  10.1.1.0 255.255.255.0

C.  10.1.1.30 255.255.255.255

D.  10.1.1.30 0.0.0.255

Q37. What happens if there is a native VLAN mismatch and CDP is enabled?

A.  CDP may generate a mismatch warning.

B.  The trunk automatically corrects the mismatch.

C.  DTP disables the trunk.

D.  VLANs synchronise automatically.

Q38. Which command verifies HSRP state?

A.  show standby brief

B.  show hsrp neighbors

C.  show fhrp state

D.  show virtual gateway

Q39. A frame with an unknown destination MAC arrives on a switch. What does the switch do?

A.  Drops the frame

B.  Floods it out all ports in the VLAN except the incoming port

C.  Sends it to the root bridge only

D.  Sends it to the default gateway

Q40. Which NAT translation persists permanently?

A.  PAT

B.  Dynamic NAT

C.  Static NAT

D.  Overload NAT

Q41. A static route is configured: ip route 0.0.0.0 0.0.0.0 10.1.1.2 200. What makes it floating?

A.  It is a default route.

B.  The next-hop IP

C.  AD 200

D.  The /0 mask

Q42. Which protocol uses multicast address 224.0.0.5?

A.  IGMP

B.  OSPF

C.  RIP

D.  HSRP

Q43. Which two items must match for OSPF neighbours to form?

A.  Area ID and hello/dead timers

B.  Router ID and bandwidth

C.  Cost and hostname

D.  MTU and default route

Q44. Which DHCP message does the client send first?

A.  Offer

B.  Request

C.  Discover

D.  ACK

Q45. Which command shows the MAC address table?

A.  show arp

B.  show mac address-table

C.  show switching

D.  show interfaces mac

Q46. Routes: 10.0.0.0/8, 10.1.0.0/16, 10.1.1.0/24. The destination is 10.1.1.200. Which route has the longest prefix match?

A.  /8

B.  /16

C.  /24

D.  Equal cost

Q47. What is true about switchport mode dynamic auto?

A.  It forces a trunk.

B.  It is a passive DTP mode.

C.  It disables DTP.

D.  It creates a permanent access port.

Q48. Which attack does DHCP snooping help prevent?

A.  Rogue DHCP server

B.  VLAN hopping

C.  MAC flooding

D.  BPDU spoofing

Q49. A switch has two equal-cost Layer 2 paths. Which STP rule is used first to choose the designated port?

A.  Lowest bandwidth

B.  Lowest root path cost

C.  Highest MAC

D.  Lowest LAN ID

Q50. A packet is sent to 192.168.1.255 on a 192.168.1.0/24 network. What type is it?

A.  Directed broadcast

B.  Limited broadcast

C.  Multicast

D.  Loopback

Q51. Users in VLAN 20 cannot reach the default gateway. Access ports are in VLAN 20, the trunk allows VLAN 20, and interface VLAN 20 (10.20.20.1/24) exists but is shut down. What is the root cause?

A.  VLAN mismatch

B.  Incorrect subnet mask

C.  SVI administratively down

D.  Trunking failure

Q52. SW1 uses channel-group 1 mode active and SW2 uses channel-group 1 mode on. Why does the EtherChannel fail?

A.  Active requires passive or active, not on

B.  LACP and static modes mismatch

C.  VLAN mismatch

D.  Port security conflict

Q53. OSPF neighbours are stuck in Init. What is most likely?

A.  Duplicate router ID

B.  Hello packets received one way only

C.  MTU mismatch

D.  Area mismatch after full state

Q54. Hosts can ping the local subnet but not remote networks. The routing table shows the gateway of last resort is not set. What is the likely cause?

A.  Missing default route

B.  ARP poisoning

C.  STP loop

D.  Native VLAN mismatch

Q55. A switch suddenly has very high CPU and CAM table instability. What is the likely attack?

A.  VLAN hopping

B.  DHCP starvation

C.  MAC flooding

D.  BPDU spoofing

Q56. OSPF adjacency will fail if which mismatch exists?

A.  Router hostname

B.  MTU

C.  Interface description

D.  IOS version

Q57. Hosts receive 169.254.x.x addresses. What is the most likely cause?

A.  Duplicate IP conflict

B.  DHCP failure

C.  Incorrect DNS server

D.  Trunk native VLAN mismatch

Q58. An engineer configures access-list 10 deny 10.1.1.5 then access-list 10 permit any. What is the result?

A.  Invalid ACL syntax

B.  Host denied, others permitted

C.  Entire subnet denied

D.  ACL ignored

Q59. Users report intermittent duplicate packets in a switched network. What is the most likely cause?

A.  STP disabled, creating a loop

B.  DHCP snooping issue

C.  PortFast enabled

D.  OSPF passive interface

Q60. A router has an OSPF route (AD 110), a static route (AD 1) and a floating static route (AD 200) to the same destination. OSPF fails. Which route is used?

A.  Floating static

B.  No route

C.  Static AD 1

D.  Both

Q61. Inter-VLAN routing fails with router-on-a-stick. The sub-interface has encapsulation dot1q 10 and ip address 10.10.10.1 255.255.255.0. The switch trunk uses native VLAN 10. What is the problem?

A.  IP is incorrect

B.  The sub-interface should use encapsulation dot1q 10 native

C.  OSPF is missing

D.  HSRP is needed

Q62. What happens when BPDU Guard triggers?

A.  The port goes to listening

B.  The port goes err-disabled

C.  The port becomes a root port

D.  The port changes VLAN

Q63. A ping works by IP address but not by hostname. What is the likely issue?

A.  DHCP

B.  DNS

C.  STP

D.  DNAT

Q64. OSPF neighbours are Full but routes are not exchanged. What is the most likely cause?

A.  Passive interface

B.  Duplicate MAC

C.  Port security

D.  ARP timeout

Q65. A trunk passes VLANs 10 and 20. VLAN 30 suddenly fails. Why?

A.  VLAN 30 is not allowed on the trunk

B.  Native VLAN issue

C.  DTP failure

D.  CDP disabled

Q66. Users lose connectivity after a rogue switch is plugged into an access port. What is the best protection?

A.  DHCP snooping

B.  Port security plus BPDU Guard

C.  NAT overload

D.  HSRP

Q67. A traceroute stops at hop 4. What did the hop 4 device most likely do?

A.  Blocked ARP

B.  Let TTL expire and sent ICMP

C.  Disabled OSPF

D.  Changed VLAN

Q68. A switch receives a superior BPDU unexpectedly on an access port. What does this indicate?

A.  A rogue switch is likely connected

B.  DHCP starvation

C.  OSPF issue

D.  EtherChannel mismatch

Q69. Users can browse the internet but an internal private server is unreachable after NAT is configured. What is the likely cause?

A.  NAT translation affecting internal traffic

B.  OSPF hello mismatch

C.  Port security violation

D.  CDP disabled

Q70. A LACP bundle forms but traffic is black-holed. What is most likely?

A.  MTU mismatch across bundled links

B.  ACL logging enabled

C.  CDP disabled

D.  Router ID mismatch

Q71. What causes OSPF neighbours to stay in 2-Way on Ethernet when Full is expected?

A.  Both routers are DR

B.  Router priority 0 on a DROther relationship

C.  ACL deny

D.  Wrong MAC address

Q72. A host sends a frame to a remote network destination. The first Layer 2 destination is:

A.  The final remote host's MAC

B.  The default gateway's MAC

C.  The broadcast MAC

D.  The root bridge MAC

Q73. You see %CDP-4-NATIVE_VLAN_MISMATCH. What is the immediate risk?

A.  Routing loop

B.  VLAN leakage

C.  OSPF failure

D.  DHCP starvation

Q74. An ACL intended to block Telnet is applied outbound instead of inbound. What is the result?

A.  ACL ignored

B.  Telnet may still enter the network

C.  The router reloads the ACL

D.  Traffic encrypted

Q75. The network converges slowly after a link failure. What is the most likely Layer 2 cause?

A.  High STP timers

B.  DNS failure

C.  DHCP relay issue

D.  Port security aging

Q76. A switch has switchport mode dynamic desirable and switchport nonegotiate. What is the effect?

A.  The port actively negotiates trunking

B.  DTP frames are suppressed despite desirable mode

C.  The port becomes access by default

D.  The switch falls back to dynamic auto

Q77. Which IPv6 address type can be assigned to multiple devices to reach the nearest instance?

A.  Link-local

B.  Multicast

C.  Anycast

D.  Unique local

Q78. An OSPF route has metric 20. A second OSPF route to the same prefix also has metric 20 via another next hop. What happens?

A.  Lower router ID wins

B.  Lower next-hop IP wins

C.  Equal-cost load balancing may occur

D.  The first learned route is the only one kept

Q79. Which command prevents an access port from becoming a trunk if a rogue switch sends DTP?

A.  switchport mode access

B.  switchport nonegotiate

C.  spanning-tree bpduguard enable

D.  ip dhcp snooping trust

Q80. A router receives a packet for 10.10.10.10. It has 10.10.10.0/24 via OSPF and 10.10.10.0/24 via a static route with AD 200. Which is used?

A.  Static, because it is manual

B.  OSPF, due to lower AD

C.  Equal-cost load balancing

D.  Longest prefix causes a drop

Q81. Which condition can make a PortFast-enabled access port unsafe?

A.  Connected to a printer

B.  Connected to an end host

C.  Connected to another switch

D.  Connected to a phone

Q82. What does OSPF use as its metric?

A.  Delay

B.  Hop count

C.  Cost based on bandwidth

D.  Composite metric

Q83. Which wildcard mask matches subnet 192.168.8.0/21?

A.  0.0.7.255

B.  0.0.3.255

C.  255.255.248.0

D.  0.0.15.255

Q84. An interface shows administratively down, line protocol down. What is the most likely cause?

A.  Cable unplugged

B.  shutdown applied

C.  Speed mismatch

D.  Encapsulation mismatch

Q85. Which HSRP router answers ARP for the virtual IP?

A.  Standby only

B.  Active router

C.  Both active and standby

D.  Router with the highest interface IP

Q86. Which statement about ip default-gateway on a Layer 2 switch is true?

A.  Used only when IP routing is disabled

B.  Replaces static routes

C.  Required for VLAN trunking

D.  Used by OSPF

Q87. Which feature helps prevent VLAN hopping via double tagging?

A.  Use a dedicated native VLAN and keep user ports out of it

B.  Enable PortFast

C.  Disable ARP inspection

D.  Use dynamic desirable

Q88. A switch receives a frame destined to its own management SVI. How is it processed?

A.  Layer 1 only

B.  By the control plane CPU

C.  By the root bridge

D.  By the CAM aging process

Q89. Which is true about RSTP compared to classic STP?

A.  Uses distance-vector logic

B.  Faster convergence

C.  Removes root bridge election

D.  Requires no BPDUs

Q90. A DHCP client gets an IP address but the wrong default gateway. What is the most likely issue?

A.  Option 3 misconfigured on the DHCP server

B.  TTL expired

C.  ARP cache full

D.  MTU mismatch

Q91. Which command verifies which switch is the root bridge?

A.  show spanning-tree root

B.  show spanning-tree

C.  show vlan root

D.  show stp bridge

Q92. What happens if an ACL has no explicit permit and nothing matches?

A.  Traffic is permitted by default

B.  The implicit deny drops traffic

C.  The router asks for confirmation

D.  ACL ignored

Q93. Which OSPF state indicates the adjacency is complete?

A.  Exchange

B.  ExStart

C.  Full

D.  Init

Q94. A switch sees the same source MAC moving rapidly between ports. This suggests:

A.  HSRP failover

B.  MAC flapping or a possible loop

C.  DNS issue

D.  DHCP relay problem

Q95. Which address does a host ARP for when sending to a remote subnet?

A.  Remote host IP

B.  Remote host MAC

C.  Default gateway MAC

D.  DNS server MAC

Q96. What does passive-interface do in OSPF?

A.  Stops Hellos being sent on the interface

B.  Deletes the interface IP

C.  Blocks route installation

D.  Removes the area assignment

Q97. Which command displays learned secure MAC addresses with port security?

A.  show port-security address

B.  show secure mac

C.  show switchport mac security

D.  show mac sticky

Q98. Which protocol uses IP protocol number 89 rather than a TCP/UDP port?

A.  EIGRP

B.  OSPF

C.  SNMP

D.  SSH

Q99. A trunk allows VLANs 10, 20 and 30. VTP adds VLAN 40 to the database. What happens across this trunk?

A.  VLAN 40 passes automatically

B.  VLAN 40 is blocked unless allowed on the trunk

C.  The trunk resets

D.  STP disables VLAN 10

Q100. Which issue most likely causes intermittent duplex mismatch symptoms?

A.  Late collisions and poor performance

B.  OSPF stuck in Init

C.  Native VLAN mismatch logs only

D.  DHCP scope exhaustion