CCNA Exam v1.1 (200-301)

Complete Integrated Study Notes

# Part 1 — Networking Foundations

📋 Exam ref: 1.0 — Network Fundamentals — 20% of exam

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| IEEE | Institute of Electrical and Electronics Engineers | Professional association that develops networking and technology standards. |
| IETF | Internet Engineering Task Force | Organisation that develops and promotes voluntary Internet standards. |
| UTP | Unshielded Twisted Pair | Ethernet cabling consisting of twisted wire pairs without shielding. |
| SFP | Small Form-Factor Pluggable | Compact, hot-swappable transceiver used for fibre or copper connections. |
| LAN | Local Area Network | Network of devices in a single broadcast domain, typically in one building. |
| SOHO | Small Office / Home Office | Small-scale business or home office network. |
| NGFW | Next-Generation Firewall | Advanced firewall with deep packet inspection, application awareness, and IPS. |
| NGIPS | Next-Generation Intrusion Prevention System | Security system that detects and prevents network intrusions in real-time. |
| CDP | Cisco Discovery Protocol | Cisco proprietary protocol that discovers directly connected Cisco devices. |

## Network Components

📋 Exam ref: 1.1 — Explain the role and function of network components

### Routers

📋 Exam ref: 1.1.a — Routers

- Routers forward packets between different networks using IP addresses (Layer 3).

- Each interface on a router connects to a different network/subnet.

- Routers use their routing table to determine the best path for each packet.

### Switches

📋 Exam ref: 1.1.b — Layer 2 and Layer 3 switches

- Layer 2 switch — forwards frames using MAC addresses. Operates within a single network/VLAN.

- Layer 3 switch (multilayer switch) — can also route between VLANs using SVIs. Faster than router-on-a-stick.

### Next-Generation Firewalls and IPS

📋 Exam ref: 1.1.c — Next-generation firewalls and IPS

- Firewall — monitors and filters traffic based on rules, protecting the network from unauthorised access.

- NGFW — adds deep packet inspection, application awareness, and intrusion prevention to traditional firewalls.

- IPS (Intrusion Prevention System) — monitors traffic for malicious activity and actively blocks threats in real time.

- IDS (Intrusion Detection System) — detects/alerts only — does not block traffic.

### Other Network Components

📋 Exam ref: 1.1.d-h — Access points, Controllers, Endpoints, Servers, PoE

- Access Point (AP) — provides wireless connectivity to the wired network.

- WLC (Wireless LAN Controller) — centrally manages multiple APs.

- Endpoints — devices that connect to the network: PCs, laptops, phones, printers, cameras.

- Servers — provide services to clients (web, file, email, DNS, DHCP).

- PoE (Power over Ethernet) — carries electrical power to devices like APs, IP phones, and cameras over the network cable.

## Network Topology Architectures

📋 Exam ref: 1.2 — Network topology architectures

- Two-tier (Collapsed Core) — Access + Distribution/Core combined. Used in smaller networks.

[PC]  [PC]  [PC]  [PC]

|     |     |     |

[Access Switch]  [Access Switch]

|                |

+------+----------+

|

[Distribution/Core Switch]   <- layers combined

|

[Router]

- Three-tier — Access → Distribution → Core. Used in large enterprise networks.

[PC] [PC]     [PC] [PC]

|   |         |   |

[Access SW]  [Access SW]  <- Tier 1: Access

|              |

+------+-------+

|

[Distribution SW]         <- Tier 2: Distribution

|

[Core SW]             <- Tier 3: Core

|

[Router/WAN]

- Spine-leaf — data centre design. Every leaf connects to every spine. Predictable latency.

[Spine 1]--------[Spine 2]  <- all spines link together

/   |   \      /   |   \

[L1] [L2] [L3] [L4] [L5] [L6] <- Leaf switches

|              |

[Servers]     [Servers]

Every leaf connects to every spine.

Traffic = always 2 hops. Predictable latency.

- WAN — connects geographically separated networks using leased lines, MPLS, broadband, or cellular.

[Office A]              [Office B]

|                       |

[Router A]===WAN Link===[Router B]

|

(MPLS / Leased Line / Broadband / Cellular)

- SOHO — small network typically using a single combined router/switch/AP device.

[ISP / Modem]

|

[Router + Switch + AP]   <- all-in-one SOHO device

|         |         |         |

[PC]    [Laptop]  [Printer]  [Wi-Fi Devices]

[PC]  [Laptop]  [Printer]  ))) Wi-Fi devices

- On-premises vs Cloud — on-prem = owned locally. Cloud = hosted by a provider. Hybrid = mix of both.

On-premises:  [Your building] --> [Your own servers]

Cloud:        [Your building] --> [Internet] --> [Provider servers]

Hybrid:       [Your building] --> [Your servers]

|                     +

+-------> [Internet] --> [Provider servers]

## Interfaces and Cabling

📋 Exam ref: 1.3 — Compare physical interface and cabling types

- Ethernet is a collection of network protocols and standards (IEEE 802.3, published 1983).

- 8 Bits = 1 Byte. Speed measured in bits per second.

- Auto-MDIX eliminates the need for crossover cables by automatically detecting and configuring TX/RX pins.

### Ethernet Speeds

| Speed | Name | Standard | Cable Type | Max Distance |
| --- | --- | --- | --- | --- |
| 10 Mbps | Ethernet | 802.3i | 10Base-T | 100m |
| 100 Mbps | Fast Ethernet | 802.3u | 100Base-T | 100m |
| 1000 Mbps | Gigabit Ethernet | 802.3ab | 1000Base-T | 100m |
| 10 Gbps | 10 Gigabit Ethernet | 802.3an | 10GBase-T | 100m |

### UTP Cables

- 10Base-T and 100Base-T = 2 pairs (4 wires)

- 1000Base-T and 10GBase-T = 4 pairs (8 wires)

- Full Duplex — both send and receive at the same time.

| Connection | Cable Type |
| --- | --- |
| Router to Switch | Straight-through |
| Switch to Switch | Crossover |
| Router to Router | Crossover |
| Switch to PC | Straight-through |

### Fibre

📋 Exam ref: 1.3.a — Single-mode fiber, multimode fiber, copper

- SFP transceiver — Small Form-Factor Pluggable. Connects fibre to network devices.

- Each fibre is unidirectional: one TX, one RX — connections cross over.

- Single Mode — smallest core, longest distance, most expensive. Yellow jacket.

- Multi-Mode — wider core, shorter than single mode but longer than UTP. Orange/aqua jacket.

### Interface and Cable Issues

📋 Exam ref: 1.4 — Identify interface and cable issues

- Duplex mismatch — one side full duplex, other half duplex. Causes collisions and poor performance.

- Speed mismatch — interfaces negotiated to different speeds.

- Collisions — occur in half-duplex when two devices transmit simultaneously.

- Errors — CRC/FCS errors shown in show interfaces. Often caused by bad cable, connector, or duplex mismatch.

⌨  Key Commands for this section

| Command | Purpose | Description | Device |
| --- | --- | --- | --- |
| show interfaces | View interface detail | Shows Layer 1/2 status, speed, duplex, MAC address, and error counters. | Both |
| show version | View system info | Provides details about IOS version, memory usage, and uptime. | Both |

# Part 2 — The TCP/IP Model

📋 Exam ref: 1.5 — Compare TCP to UDP

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| TCP | Transmission Control Protocol | Reliable, connection-oriented Layer 4 transport protocol guaranteeing delivery and ordering. |
| UDP | User Datagram Protocol | Fast, connectionless Layer 4 transport protocol with no delivery guarantee. |
| IP | Internet Protocol | Main Layer 3 protocol responsible for addressing and routing packets. |
| HTTP | Hypertext Transfer Protocol | Layer 5 protocol used by web browsers and servers to transfer web pages. |
| HTTPS | Hypertext Transfer Protocol Secure | Encrypted version of HTTP using TLS to secure data in transit. |
| ICMP | Internet Control Message Protocol | Used for error reporting and diagnostics (e.g. ping). |
| PDU | Protocol Data Unit | Name for data at each layer: bits (L1), frames (L2), packets (L3), segments (L4). |
| IEEE | Institute of Electrical and Electronics Engineers | Professional association that develops networking and technology standards. |
| IETF | Internet Engineering Task Force | Organisation that develops and promotes voluntary Internet standards. |
| PING | Packet Internet Groper | Network utility used to test the reachability of a host on an IP network. |

## Protocols and Standards

- Protocol — a set of rules defining how data is communicated between devices.

- IEEE — defines LAN technology standards (Ethernet, Wi-Fi).

- IETF — defines internet protocols (TCP, UDP, HTTP, DNS, etc.)

- Each layer services the layer below and above. Together they form the Network Stack.

## TCP/IP Model Overview

| Layer | Name | Identifies By | Examples |
| --- | --- | --- | --- |
| Layer 1 | Physical | Bits / signals | Cables, fibre, radio |
| Layer 2 | Local Network | MAC addresses | Ethernet, Wi-Fi |
| Layer 3 | Internet | IP addresses | IP, ICMP |
| Layer 4 | Transport | Port numbers | TCP, UDP |
| Layer 5 | Application | Application data | HTTP, DNS, FTP |

Note: TCP/IP model = 5 layers (CCNA hybrid). OSI model = 7 layers. Application layer (TCP/IP L5) = Layers 5–7 in OSI.

### TCP vs UDP

📋 Exam ref: 1.5 — Compare TCP to UDP

- TCP — connection-oriented, reliable, ordered. Uses 3-way handshake (SYN, SYN-ACK, ACK). Used for HTTP, FTP, email.

- UDP — connectionless, no delivery guarantee, no ordering. Faster. Used for VoIP, video streaming, DNS.

Key difference: TCP = reliable but slower. UDP = fast but no guarantee. Choose based on whether the application can tolerate lost data.

| Feature | TCP | UDP |
| --- | --- | --- |
| Connection | Connection-oriented | Connectionless |
| Reliability | Guaranteed delivery | No guarantee |
| Speed | Slower (overhead) | Faster |
| Ordering | In-order delivery | No ordering |
| Uses | HTTP, FTP, email | VoIP, DNS, streaming |

Encapsulation: As data moves down the stack, each layer adds a header. Layer 2 also adds a trailer. The reverse (removing headers) is de-encapsulation.

⌨  Key Commands for this section

| Command | Purpose | Description | Device |
| --- | --- | --- | --- |
| ping <IP-address> | Test reachability | Sends ICMP Echo Requests and measures round-trip time to a destination IP. | Both |

# Part 3 — Ethernet Frames & MAC Addresses

📋 Exam ref: 1.13 — Describe switching concepts

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| PDU | Protocol Data Unit | Name for data at each layer: bits (L1), frames (L2), packets (L3), segments (L4). |
| SFD | Start Frame Delimiter | 1-byte Ethernet header field marking the end of the preamble and start of the frame. |
| FCS | Frame Check Sequence | Value in the Ethernet frame trailer used to detect transmission errors. |
| CRC | Cyclic Redundancy Check | Error-detecting code used to detect accidental changes to data (relates to FCS). |
| MAC | Media Access Control | Unique hardware identifier assigned to network interfaces. |
| OUI | Organisationally Unique Identifier | First 3 bytes of a MAC address identifying the manufacturer (assigned by IEEE). |
| BIA | Burned-In Address | Another name for a MAC address — permanently programmed into a device's NIC. |
| MTU | Maximum Transmission Unit | Largest size (bytes) of a packet/frame that can be transmitted over an interface. |

## Ethernet Frame Structure

- Ethernet frame = Ethernet header + packet + trailer

### Header Fields

- Preamble — 7 bytes (56 bits). Alternating 1s/0s. Synchronises receiver clocks.

- SFD (Start Frame Delimiter) — 1 byte (10101011). Marks end of preamble, start of frame.

- Destination — MAC address of receiving device.

- Source — MAC address of sending device.

- Type/Length — ≤1500 = length of encapsulated packet. ≥1536 = type (IPv4 = 0x0800, IPv6 = 0x86DD).

- FCS (Frame Check Sequence) — trailer. 4 bytes. Used to detect transmission errors.

Memory hook: Think of the frame like an envelope: Preamble = "attention signal", SFD = "here it comes", Destination + Source = the addresses on the envelope, Type/Length = what's inside, Payload = the letter, FCS = wax seal to verify it wasn't tampered with.

- Why Preamble/SFD exist: network hardware needs a moment to sync up before reading data. The alternating 1s/0s let the receiver lock onto the signal's timing before the actual frame data begins.

- Type/Length decision rule: if the value is 1500 or less → it's the payload length in bytes. If 1536 (0x0600) or greater → it identifies the Layer 3 protocol. In practice you'll nearly always see 0x0800 (IPv4) or 0x86DD (IPv6).

- FCS / CRC: the sender runs a mathematical calculation on the frame data and puts the result in the FCS field. The receiver runs the same calculation — if the result doesn't match, the frame is silently dropped.

Frame size: Max Ethernet frame = 1518 bytes (1522 with VLAN tag). Preamble+SFD+Dst+Src+Type+FCS = 26 bytes overhead.

## MAC Addresses

- 6 bytes (48 bits) — globally unique hardware address. Also called BIA (Burned-In Address).

- First 3 bytes = OUI (manufacturer). Last 3 bytes = unique to the device.

- Written as 12 hexadecimal characters (0–9, A–F).

| Hex | Decimal | Hex | Decimal |
| --- | --- | --- | --- |
| 0 | 0 | 8 | 8 |
| 1 | 1 | 9 | 9 |
| 2 | 2 | A | 10 |
| 3 | 3 | B | 11 |
| 4 | 4 | C | 12 |
| 5 | 5 | D | 13 |
| 6 | 6 | E | 14 |
| 7 | 7 | F | 15 |

### Frame Types and MAC Learning

📋 Exam ref: 1.13.a-d — MAC learning and aging, frame switching, frame flooding, MAC address table

- Unicast — destined for one device.

- Unknown unicast — destination MAC unknown to switch → flooded to all ports except source.

- Known unicast — destination MAC known → forwarded only to that port.

- Broadcast — destination FFFF.FFFF.FFFF. Sent to all hosts in the broadcast domain.

- Dynamic MAC addresses — learned automatically. Aged out after 5 minutes of inactivity.

⌨  Key Commands for this section

| Command | Purpose | Description | Device |
| --- | --- | --- | --- |
| show mac address-table | View MAC address table | Lists MAC addresses learned by the switch, with VLAN, type (static/dynamic), and port. | Switch |

# Part 4 — Ethernet LAN Switching

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| ARP | Address Resolution Protocol | Resolves a known IP address to its MAC address in a local network. |
| ICMP | Internet Control Message Protocol | Used for error reporting and diagnostics (e.g. ping). |
| PING | Packet Internet Groper | Network utility used to test the reachability of a host on an IP network. |
| MAC | Media Access Control | Unique hardware identifier assigned to network interfaces. |

## ARP (Address Resolution Protocol)

- Resolves a known IP address to its MAC address on the local network.

- ARP Request — broadcast (FFFF.FFFF.FFFF). "Who has IP x.x.x.x?"

- ARP Reply — unicast back to the requester. "I have it, my MAC is xxxx."

## Ping

- Tests reachability and measures round-trip time.

- Uses ICMP Echo Request and ICMP Echo Reply.

Important: ARP must run before ping if the MAC address is unknown. The device sends ARP first to get the MAC, then sends the ICMP.

## MAC Address Table

- Output columns: VLAN | MAC Address | Type (static/dynamic) | Ports

⌨  Key Commands for this section

| Command | Purpose | Description | Device |
| --- | --- | --- | --- |
| arp -a | Display ARP table | Shows IP-to-MAC address mappings, including type (static or dynamic) and interface. | Both |
| ping <IP-address> | Test reachability | Sends ICMP Echo Requests and measures round-trip time to a destination IP. | Both |
| show arp | Display ARP table | Lists the ARP table entries on the device. | Both |
| show mac address-table | View MAC address table | Lists MAC addresses learned by the switch, with VLAN, type, and port. | Switch |

# Part 5 — CLI Basics

📋 Exam ref: 5.3 — Configure and verify device access control using local passwords

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| NVRAM | Non-Volatile Random Access Memory | Memory on a Cisco device retaining the startup config even when powered off. |
| AAA | Authentication, Authorization, Accounting | Security framework controlling access, tracking activity, and logging usage. |
| RADIUS | Remote Authentication Dial-In User Service | Protocol for centralised authentication, authorisation, and accounting. |
| TACACS+ | Terminal Access Controller Access-Control System Plus | Cisco-preferred AAA protocol that separates authentication, authorisation, and accounting. |

## Command Prompt Modes

| Prompt | Mode | What you can do |
| --- | --- | --- |
| Router> | User EXEC | Very limited — view only, no changes. |
| Router# | Privileged EXEC | Full visibility. Enter with enable. Save config, restart device. |
| Router(config)# | Global config | Full config changes. Enter with configure terminal. |

## Configuration Files

- Running config — active config. Lost on restart if not saved.

- Startup config — loaded on boot. Stored in NVRAM. Persists after power off.

- NVRAM — Non-Volatile RAM. Holds startup config even when device is powered off.

## Passwords

- Always use enable secret — not enable password. Uses MD5 encryption.

- If both are configured, enable password is ignored.

- service password-encryption — encrypts plaintext passwords in the config file. Has NO effect on enable secret (always encrypted).

## SSH — Secure Remote Access

📋 Exam ref: 2.8 / 4.8 — Network device management access / Configure SSH

- Telnet — sends data in plaintext. Not secure. Port 23. Avoid.

- SSH (Secure Shell) — encrypted remote access. Always preferred. Port 22.

⌨  Key Commands for this section

| Command | Purpose | Description | Device |
| --- | --- | --- | --- |
| enable | Enter privileged EXEC mode | Moves from user EXEC (>) to privileged EXEC (#). Required before making changes. | Both |
| exit | Exit current mode | Steps back one level in the CLI hierarchy. Also closes a Telnet/SSH session. | Both |
| end | Return to privileged EXEC | Jumps directly back to privileged EXEC (#) from any config sub-mode. | Both |
| configure terminal | Enter global config mode | Allows access to configuration modes where device settings can be changed. | Both |
| hostname <name> | Set device hostname | Assigns a unique name to the device for identification. | Both |
| enable secret <password> | Secure privileged EXEC | Sets an MD5-encrypted password to protect access to privileged EXEC mode. | Both |
| service password-encryption | Encrypt plaintext passwords | Encrypts all cleartext passwords in config. NO effect on enable secret. | Both |
| copy running-config startup-config | Save config to NVRAM | Saves the running configuration so changes persist after a reboot. | Both |
| show running-config | View active configuration | Displays the current running configuration on the device. | Both |
| show version | View system information | Provides IOS version, memory, uptime, and processor details. | Both |

## Keyboard Shortcuts

| Shortcut | Function |
| --- | --- |
| Up Arrow | Cycle through previously entered commands |
| ? | Context-sensitive help — lists available commands |
| Ctrl+A | Move cursor to beginning of line |
| Ctrl+E | Move cursor to end of line |
| Ctrl+K | Delete from cursor to end of line |
| Ctrl+X | Delete from cursor to beginning of line |
| Ctrl+Shift+6 | Interrupt a long-running command |
| Ctrl+C | Exit configuration mode / return to privileged EXEC |

Reference: See the Basic Configuration Commands document for the complete command list.

# Part 6 — IPv4 Addressing

📋 Exam ref: 1.6 — Configure and verify IPv4 addressing and subnetting

📋 Exam ref: 1.7 — Describe private IPv4 addressing

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| IP | Internet Protocol | Main Layer 3 protocol responsible for addressing and routing packets. |
| IANA | Internet Assigned Numbers Authority | Body that originally allocated IP address blocks to organisations. |
| CIDR | Classless Inter-Domain Routing | Method for allocating IP addresses, replacing the older classful system. |
| NAT | Network Address Translation | Remaps private IP addresses to a public IP, allowing multiple devices to share one public IP. |
| PAT | Port Address Translation | Form of NAT mapping multiple private IPs to one public IP using different port numbers. |

## IPv4 Basics

- 32 bits long (4 bytes). Dotted decimal notation (e.g. 192.168.1.1).

- Each octet = 8 bits, max value 255.

- Class D — multicast (224.0.0.0–239.255.255.255)

- Class E — reserved/experimental

- 127.0.0.0/8 — loopback range. Tests local network stack.

- 169.254.0.0/16 — APIPA (Automatic Private IP Addressing). Assigned automatically when a device fails to get an address from DHCP.

  - APIPA range: 169.254.0.1 – 169.254.255.254. Device self-assigns a random address in this range.

  - NOT routable — only works on the local segment. No internet or inter-network access.

  - If you see 169.254.x.x on a device — DHCP has failed. Check: Is DHCP server reachable? Is the cable connected? Is the DHCP scope full?

Critical: Network address = first address in the block (cannot be assigned). Broadcast = last address (cannot be assigned). Usable hosts = 2^(host bits) − 2.

### Private IPv4 Ranges

📋 Exam ref: 1.7 — Describe private IPv4 addressing

| Class | Private Range | Prefix | Typical Use |
| --- | --- | --- | --- |
| A | 10.0.0.0 – 10.255.255.255 | /8 | Large enterprises |
| B | 172.16.0.0 – 172.31.255.255 | /12 | Medium networks |
| C | 192.168.0.0 – 192.168.255.255 | /16 | Home / SOHO networks |

## Interface Status Commands

⌨  Key Commands for this section

| Command | Purpose | Description | Device |
| --- | --- | --- | --- |
| show ip interface brief | Quick interface overview | Shows interface name, IP, OK?, method, Layer 1 status, Layer 2 protocol status. | Both |
| show interfaces | Detailed interface information | Shows Layer 1/2 status, MAC address, speed, duplex, and error counters. | Both |
| show interfaces description | Interface status and description | Lists each interface with status, protocol state, and configured description. | Both |
| ip address <IP> <mask> | Assign IP to interface | Configures the IP address and subnet mask on a specific interface. | Both |
| no shutdown | Enable an interface | Activates the interface (router interfaces are shutdown by default). | Both |
| shutdown | Disable an interface | Deactivates the interface, stopping all traffic. | Both |
| description <text> | Add interface description | Adds a label to an interface for network management and troubleshooting. | Both |

# Part 7 — IPv4 Header & QoS

📋 Exam ref: 4.7 — QoS classification, marking, queuing, congestion, policing, shaping

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| DSCP | Differentiated Services Code Point | IP header field used by QoS to classify and prioritise different traffic types. |
| QoS | Quality of Service | Techniques that prioritise traffic to ensure performance for delay-sensitive applications. |
| EF | Expedited Forwarding | DSCP class for real-time traffic like VoIP — highest priority forwarding. |
| AF | Assured Forwarding | DSCP class providing varying levels of priority for non-real-time traffic. |
| BE | Best Effort | Default DSCP class for non-prioritised traffic — no QoS guarantees. |
| VoIP | Voice over IP | Technology for delivering voice communications over IP networks. |
| CoS | Class of Service | 3-bit field in 802.1q tags used to prioritise network traffic at Layer 2. |
| MTU | Maximum Transmission Unit | Largest size (bytes) of a packet/frame that can be transmitted over an interface. |

## IPv4 Header Fields

- Version — 4 bits. IPv4 = 0100, IPv6 = 0110.

- IHL (Internet Header Length) — 4 bits. Min = 5 (20 bytes), max = 15 (60 bytes).

- DSCP — 6 bits. Used for QoS traffic prioritisation.

- TTL (Time to Live) — decremented by 1 at each router hop. Packet dropped at 0. Prevents routing loops.

## QoS and DSCP

| Class | Name | Used for | Priority |
| --- | --- | --- | --- |
| EF | Expedited Forwarding | VoIP, video conferencing — real-time | Highest |
| AF | Assured Forwarding | Priority traffic (varies) | Medium |
| BE | Best Effort | Default non-prioritised traffic | Lowest |

⌨  Key Commands for this section

| Command | Purpose | Description | Device |
| --- | --- | --- | --- |
| bandwidth <value> | Set interface bandwidth | Configures the bandwidth value used by routing protocols and QoS for cost calculations. | Both |
| mtu <size> | Set Maximum Transmission Unit | Sets the maximum packet size (bytes) that can be transmitted through the interface. | Both |

# Part 8 — Subnetting

📋 Exam ref: 1.6 — Configure and verify IPv4 addressing and subnetting

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| CIDR | Classless Inter-Domain Routing | Method for allocating IP addresses, replacing the older classful system. |
| IANA | Internet Assigned Numbers Authority | Body that originally allocated IP address blocks to organisations. |

## CIDR (Classless Inter-Domain Routing)

- IANA originally assigned IPs by class (A/B/C). Led to many wasted addresses.

- IETF introduced CIDR in 1993 to allow flexible subnet sizes.

- Classful addressing — the original IP system (pre-1993). Fixed subnet masks per class:

  - Class A: /8  (255.0.0.0)   — first octet 1–126

  - Class B: /16 (255.255.0.0)  — first octet 128–191

  - Class C: /24 (255.255.255.0)— first octet 192–223

- Problem: a company needing 300 hosts had to use a full Class B — wasting 65,000+ addresses.

- Classless (CIDR) — any prefix length allowed (e.g. /19, /27, /30). Sized to fit the actual need. Far more efficient.

- As network bits go up, host bits go down.

- /31 = point-to-point link (only 2 addresses). /32 = single host (Local route).

## Subnetting Cheat Sheet

### Bit Value Table — Memorise This

128  |  64  |  32  |  16  |  8  |  4  |  2  |  1

- 192 = 128+64  |  224 = 128+64+32  |  255 = all 8 bits on

### /Notation to Subnet Mask

- /24 = 24 ones in first 3 octets → 255.255.255.0

- /25 = 1 more (128) → 255.255.255.128

- /26 = 2 more (128+64) → 255.255.255.192

- /27 = 3 more (128+64+32) → 255.255.255.224

- /28 = 4 more (128+64+32+16) → 255.255.255.240

- /29 = 5 more (248) → 255.255.255.248

- /30 = 6 more (252) → 255.255.255.252

### CIDR Quick Reference

| CIDR | Mask (last octet) | Magic # | Subnets from /24 | Usable Hosts |
| --- | --- | --- | --- | --- |
| /25 | 128 | 128 | 2 | 126 |
| /26 | 192 | 64 | 4 | 62 |
| /27 | 224 | 32 | 8 | 30 |
| /28 | 240 | 16 | 16 | 14 |
| /29 | 248 | 8 | 32 | 6 |
| /30 | 252 | 4 | 64 | 2 |

Usable hosts = 2^(host bits) − 2

### Magic Number Shortcut

Magic number = 256 − last octet of mask. This is your block size (IPs per subnet including network and broadcast).

- Step 1 — Magic number: /27 → mask 224 → 256−224 = 32

- Step 2 — Count up in multiples: 0, 32, 64, 96, 128, 160, 192, 224

- Step 3 — Network = start of block. Broadcast = next block − 1. First host = Net+1. Last host = BC−1.

### Worked Example: Divide 192.168.255.0/24 into 5 Equal Subnets

- Need 5 subnets → next power of 2: 2³ = 8 ✓

- Borrow 3 bits → /24 + 3 = /27

- /27 = 255.255.255.224. Magic number = 32. Boundaries: 0, 32, 64, 96, 128, 160, 192, 224

| Subnet | Network Address | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- |
| 1 | 192.168.255.0/27 | 192.168.255.1 | 192.168.255.30 | 192.168.255.31 |
| 2 | 192.168.255.32/27 | 192.168.255.33 | 192.168.255.62 | 192.168.255.63 |
| 3 | 192.168.255.64/27 | 192.168.255.65 | 192.168.255.94 | 192.168.255.95 |
| 4 | 192.168.255.96/27 | 192.168.255.97 | 192.168.255.126 | 192.168.255.127 |
| 5 | 192.168.255.128/27 | 192.168.255.129 | 192.168.255.158 | 192.168.255.159 |

## IPv6 Addressing

📋 Exam ref: 1.8 / 1.9 — Configure and verify IPv6 addressing / Describe IPv6 address types

⚠ GAP — 1.8 / 1.9: IPv6 Addressing and Address Types  (Add notes from video when studied)

Cover: 128-bit addressing, hex notation, address types (global unicast, unique local, link local, anycast, multicast), modified EUI-64, dual stack, configuring IPv6 on interfaces.

# Part 9 — Routing

📋 Exam ref: 3.0 — IP Connectivity — 25% of exam

## RIP (Routing Information Protocol)

📋 Exam ref: 3.3 — Dynamic routing protocols (RIP overview)

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| RIP | Routing Information Protocol | Distance vector IGP that uses hop count as its metric. Max 15 hops. |

- RIP — Distance Vector IGP. Uses "routing-by-rumour" — each router only knows what its neighbours tell it.

- Metric = hop count. One router = 1 hop. Bandwidth is irrelevant.

- Maximum hop count = 15. Any destination more than 15 hops away is considered unreachable.

- Shares the full routing table with neighbours every 30 seconds. Can flood traffic on large networks.

- Two message types: Request (ask neighbour for their table) | Response (send your table to neighbours).

### RIPv1

- IPv4 only. Classful only — advertises Class A, B, and C networks. No VLSM or CIDR support.

- Does not include subnet mask in advertisements — neighbours must guess the mask from the class.

- Broadcasts to 255.255.255.255 — every device on the segment receives the update, not just RIP neighbours.

### RIPv2

- IPv4 only. Supports VLSM and CIDR — includes subnet mask in advertisements (e.g. /30).

- Multicasts to 224.0.0.9 — only RIP-enabled routers receive the update. Less noisy than broadcast.

- Broadcast vs Multicast: broadcast (255.255.255.255) goes to all devices on the segment. Multicast (224.0.0.9) goes only to RIP-enabled routers that have joined that multicast group.

### RIPng

- IPv6 only. Next Generation RIP. Same concepts as RIPv2 but for IPv6 addresses.

### The network command

- The network command is classful — it automatically converts the address you enter to a classful boundary.

- Example: network 10.0.12.0 → converted to network 10.0.0.0 (Class A /8).

- What the network command actually does:

  - Looks for interfaces with an IP in the specified range and activates RIP on them.

  - Forms adjacencies with connected RIP neighbours on those interfaces.

  - Advertises the actual interface prefix (e.g. 10.0.12.0/30) — NOT the classful address in the network command.

  - Does NOT tell the router which networks to advertise — it tells it which interfaces to activate RIP on.

Note: OSPF and EIGRP network commands operate the same way — they activate the protocol on matching interfaces.

### Passive Interface

- Stops RIP sending advertisements out of an interface that has no RIP neighbours.

- The router still advertises the prefix of that interface to its other RIP neighbours.

- Always use passive-interface on ports facing end devices or with no RIP neighbours. EIGRP and OSPF use the same command.

### show ip protocols

- Works for RIP, EIGRP, and OSPF. Shows the current state of all routing protocols on the device.

- Key output fields:

  - Automatic network summarization is not in effect — no auto-summary running.

  - Maximum path — number of equal-cost paths to load-balance across (default 4, max 32).

  - Routing for Networks — the classful networks RIP is active on.

  - Passive Interfaces — interfaces receiving but not sending RIP updates.

  - Routing information sources — known RIP neighbours.

  - Distance — Administrative Distance (120 for RIP).

### Maximum Paths (ECMP Load Balancing)

- By default RIP inserts up to 4 equal-cost paths into the routing table (ECMP — Equal Cost Multi-Path).

- Can be increased up to 32. Traffic is distributed across all equal-cost paths.

### Advertising a Default Route into RIP

- If a router has a default route to the internet, you can share it with all RIP neighbours.

- Command: default-information originate

  - Packets that don't match any specific route on neighbours will be forwarded to this router and out to the internet.

How it works: Configure: ip route 0.0.0.0 0.0.0.0 <next-hop>, then use default-information originate inside RIP config to share it.

### no auto-summary

- Prevents RIP from automatically summarising routes to classful boundaries.

- On by default in some IOS versions. Always disable it with RIPv2: no auto-summary.

⌨  Key Commands for this section

| Command | Purpose | Description | Device |
| --- | --- | --- | --- |
| router rip | Enter RIP config mode | Starts RIP router configuration mode. | Router |
| version 2 | Enable RIPv2 | Selects RIPv2 — supports VLSM/CIDR and multicast (224.0.0.9) updates. | Router |
| no auto-summary | Disable auto-summary | Prevents RIP from summarising to classful boundaries. Always use with RIPv2. | Router |
| network <classful-IP> | Advertise a network | Activates RIP on interfaces in the specified range and advertises their prefixes. | Router |
| passive-interface <int> | Stop ads on interface | Stops sending RIP updates out of the interface. Still advertises the prefix. | Router |
| default-information originate | Advertise default route | Shares the default route (0.0.0.0/0) into RIP so neighbours use this router for internet traffic. | Router |
| show ip protocols | View protocol summary | Displays networks, passive interfaces, neighbours, AD, and max paths. | Router |
| show ip rip database | View RIP database | Displays RIP-learned routes and their metrics. | Router |
| debug ip rip | Debug RIP in real time | Shows RIP updates being sent and received. | Router |

## EIGRP (Enhanced Interior Gateway Routing Protocol)

📋 Exam ref: 3.3 — Configure and verify EIGRP

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| EIGRP | Enhanced Interior Gateway Routing Protocol | Cisco-developed advanced distance vector routing protocol; now open standard. |
| AS | Autonomous System | A group of routers under a common routing policy. EIGRP neighbours must share the same AS number. |
| ECMP | Equal Cost Multi-Path | Load balancing across multiple paths with the same metric to a destination. |

- Cisco originally developed EIGRP, but has since published it openly — other vendors can now implement it.

- Advanced / hybrid distance vector protocol — combines elements of both distance vector and link-state routing.

- Much faster than RIP at reacting to topology changes (uses DUAL algorithm — Diffusing Update Algorithm).

- No 15-hop limit unlike RIP. Suitable for much larger networks.

- Multicast address: 224.0.0.10 (RIP uses 224.0.0.9 for comparison).

- ECMP load balancing across 4 equal-cost paths by default (same as RIP, configurable up to 32).

- Unequal-cost load balancing — the ONLY IGP that supports this. Uses the variance command to allow traffic across paths with different costs.

### EIGRP Metric

- Uses bandwidth and delay by default (not hop count like RIP).

- Lower bandwidth or higher delay = higher (worse) metric.

- Can also factor in load and reliability but these are not recommended for production.

### Autonomous System (AS) Number

- Configured as part of the router eigrp command: router eigrp <AS-number>

- The AS number must match on all routers that need to form EIGRP adjacencies and share routing information.

- If AS numbers differ between routers — NO adjacency will form.

### EIGRP network Command

- Works the same way as RIP and OSPF — activates EIGRP on matching interfaces, forms adjacencies, advertises prefixes.

- Unlike RIP, EIGRP can accept a classless address with a mask if you don't specify one, it assumes classful (/8, /16, /24).

- EIGRP uses a wildcard mask instead of a subnet mask in the network command.

### Wildcard Masks

- A wildcard mask is an inverted subnet mask.

- Subnet mask 1s → wildcard 0s. Subnet mask 0s → wildcard 1s.

- 0 in wildcard = "this bit MUST match"

- 1 in wildcard = "this bit doesn't matter"

Example: /24 subnet mask = 255.255.255.0 → wildcard = 0.0.0.255. Means: first 3 octets must match, last octet can be anything.

### Router ID

- EIGRP (and OSPF) use a Router ID to uniquely identify each router in the routing domain.

- Priority order for Router ID selection:

  - 1. Manually configured Router ID (most reliable — always set this explicitly).

  - 2. Highest IP address on a loopback interface (loopbacks never go down — stable choice).

  - 3. Highest IP address on any active physical interface (least preferred — can change if interface goes down).

### auto-summary

- May be enabled or disabled by default depending on IOS version.

- If enabled, disable it with no auto-summary — same as RIP.

⌨  Key Commands for this section

| Command | Purpose | Description | Device |
| --- | --- | --- | --- |
| router eigrp <AS> | Enter EIGRP config mode | Starts EIGRP and specifies the Autonomous System number. Must match on all routers. | Router |
| no auto-summary | Disable auto-summary | Prevents EIGRP from summarising to classful boundaries. Recommended. | Router |
| network <IP> <wildcard> | Advertise a network | Activates EIGRP on matching interfaces using wildcard mask. Classful if mask omitted. | Router |
| passive-interface <int> | Stop ads on interface | Stops sending EIGRP hellos out the interface. Still advertises the prefix. | Router |
| show ip eigrp neighbors | View EIGRP neighbours | Lists confirmed EIGRP adjacencies and their state. | Router |
| show ip eigrp topology | View EIGRP topology table | Shows all known routes including successors and feasible successors. | Router |
| show ip protocols | View protocol summary | Displays running routing protocols, networks, passive interfaces, AD, and max paths. | Router |
| eigrp router-id <IP> | Set Router ID manually | Explicitly sets the Router ID rather than relying on IP address selection. | Router |
| variance <multiplier> | Enable unequal-cost LB | Allows EIGRP to load-balance across paths where metric is within this multiple of the best path. | Router |

## OSPFv2 (Open Shortest Path First)

📋 Exam ref: 3.3 — Configure and verify OSPFv2 for single area

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| OSPF | Open Shortest Path First | Link-state IGP. Uses Dijkstra's SPF algorithm to calculate the shortest path to every destination. |
| LSA | Link State Advertisement | A packet OSPF routers use to share information about their directly connected links with their neighbours. |
| LSDB | Link State Database | The "map" of the network. Each router builds an identical LSDB from received LSAs, then runs SPF on it. |
| SPF | Shortest Path First | Dijkstra's algorithm. Each router independently runs SPF on the LSDB to find the best route to every destination. |
| ABR | Area Border Router | A router with interfaces in more than one OSPF area. Maintains a separate LSDB for each area. |
| ASBR | Autonomous System Boundary Router | An OSPF router connected to an external network (e.g. the internet). Uses default-information originate to share a default route into OSPF. |

- Link-state IGP — every router builds a complete "connectivity map" of the network, then independently calculates the best routes.

- Each router advertises its directly connected links to neighbours via LSAs. Those LSAs are flooded to all routers until every router has the same LSDB.

- More resource-intensive than distance vector (RIP/EIGRP) — uses more CPU and memory. But reacts faster to network changes.

- Three OSPF versions:

  - OSPFv1 (1989) — original, no longer used.

  - OSPFv2 (1998) — used for IPv4. This is what CCNA covers.

  - OSPFv3 (2008) — designed for IPv6. Can also run IPv4, but OSPFv2 is the standard for IPv4.

### LSAs and the LSDB

- Routers store network information in LSAs (Link State Advertisements), organised into the LSDB (Link State Database).

- Routers flood LSAs to neighbours until all routers in the OSPF area have an identical LSDB.

- Each LSA has a 30-minute aging timer. When it expires, the LSA is re-flooded to keep the LSDB current.

- Each router then runs the SPF algorithm on its LSDB to independently calculate the best route to every destination.

### Three Steps to OSPF Operation

- 1. Become Neighbours — establish a neighbour relationship with other routers on the same segment.

- 2. Exchange LSAs — share LSAs with neighbours until all routers have the same LSDB.

- 3. Calculate best routes — run SPF on the LSDB and insert the best routes into the routing table.

### OSPF Areas

- OSPF uses areas to divide up the network. Each area shares its own LSDB.

- Small networks can be a single area (all routers in area 0) with no performance issues.

- Large single-area designs cause problems:

  - SPF takes longer to calculate.

  - Requires exponentially more CPU and memory.

  - Larger LSDB = more memory used.

  - Any small change floods LSAs and triggers SPF re-calculation across all routers.

- Dividing into smaller areas avoids all of the above.

- Backbone area (Area 0) — all other areas must connect to Area 0.

- Internal routers — all interfaces are in the same area.

- Area Border Routers (ABRs) — interfaces in multiple areas. Maintains a separate LSDB per area. Recommended maximum: 2 areas per ABR.

- Backbone routers — any router with an interface in Area 0.

- ASBR (Autonomous System Boundary Router) — connects the OSPF network to an external network (e.g. internet).

- Intra-area route — route to a destination inside the same OSPF area.

- Interarea route — route to a destination in a different OSPF area.

Area rules: Areas must be contiguous (not split). All areas must have at least one ABR connected to Area 0. Interfaces in the same subnet must be in the same area.

### OSPF Process ID

- Configured with: router ospf <1-65535>

- The process ID is locally significant only. Routers with different process IDs CAN still form OSPF neighbours — it does not need to match.

- The process ID is completely unrelated to the area number.

- For the CCNA exam: configure single-area OSPF using area 0 only.

### The network command

- network <ip-address> <wildcard-mask> area <area-number>

- Uses a wildcard mask (same as EIGRP, not a regular subnet mask).

- What it does:

  - Finds any interfaces with an IP in the specified range.

  - Activates OSPF on those interfaces in the specified area.

  - Router then tries to form OSPF neighbour relationships on those interfaces.

Same as RIP and EIGRP: The network command activates the protocol on matching interfaces — it does not directly tell the router which networks to advertise.

### Passive Interface

- passive-interface <interface>

- Stops the router sending OSPF hello messages out of the interface — no neighbour relationships form on that interface.

- The router still advertises the subnet of that interface to its OSPF neighbours via LSAs.

- Always configure passive-interface on interfaces with no OSPF neighbours (e.g. interfaces facing end devices).

### Advertising a Default Route into OSPF

- Step 1: ip route 0.0.0.0 0.0.0.0 <ISP-IP> — configure the default route.

- Step 2: default-information originate — advertise it into OSPF.

- This causes the router to create a new LSA and flood it so all OSPF neighbours learn the default route.

- The router becomes an ASBR when default-information originate is used.

### Router ID

- Every OSPF router has a Router ID — a 32-bit value formatted like an IP address, used to uniquely identify the router.

- Priority order:

  - 1. Manually configured: router-id <ip-address> — always preferred.

  - 2. Highest IP on a loopback interface — loopbacks never go down, so this is stable.

  - 3. Highest IP on any active physical interface — least preferred, can change.

- After changing the Router ID: clear ip ospf process to apply immediately.

### show ip protocols (OSPF)

- Routing Protocol is "ospf 1" — confirms OSPF is running. The number is the process ID.

- Router ID — shows the current Router ID in use.

- Routing for Networks — shows the networks entered in the network command (which interfaces OSPF is enabled on).

- Distance: (default is 110) — OSPF Administrative Distance. Lower = more trusted.

- Maximum paths: 4 — OSPF supports ECMP load balancing over up to 4 equal-cost paths. Does NOT support unequal-cost load balancing.

⌨  Key Commands for this section

| Command | Purpose | Description | Device |
| --- | --- | --- | --- |
| router ospf <process-id> | Enter OSPF config mode | Starts OSPF. Process ID is locally significant — does not need to match between routers. | Router |
| network <ip> <wildcard> area <n> | Activate OSPF on interfaces | Activates OSPF on interfaces matching the IP range, assigns them to the specified area. | Router |
| passive-interface <int> | Stop OSPF hellos on interface | Stops sending OSPF hello messages. Router still advertises the interface subnet via LSA. | Router |
| router-id <ip-address> | Set OSPF Router ID | Manually sets the Router ID. Preferred over auto-selection from loopback or physical IPs. | Router |
| default-information originate | Advertise default route | Floods the default route (0.0.0.0/0) into OSPF. Router becomes an ASBR. | Router |
| clear ip ospf process | Reset OSPF processes | Clears and restarts all OSPF processes. Required to apply a new Router ID immediately. | Router |
| show ip ospf neighbor | View OSPF neighbours | Lists OSPF neighbours, their state, and the interface they are reachable on. | Router |
| show ip ospf | View OSPF process info | Displays Router ID, process ID, area info, and SPF algorithm statistics. | Router |
| show ip protocols | View protocol summary | Shows Router ID, networks, passive interfaces, AD (110), and max paths for OSPF. | Router |

## Routing Fundamentals

- Routing — the process of forwarding IP packets to their destination using the routing table.

- Dynamic Routing — routers share info using protocols (OSPF, EIGRP) to automatically build routing tables.

- Static Routing — manually configured routes by a network engineer.

## Route Codes in show ip route

📋 Exam ref: 3.1 — Interpret the components of routing table

- L — Local — route to the interface's own IP (/32). "This packet is for me."

- C — Connected — route to the directly connected network.

- S — Static — manually configured route.

- O — OSPF — learned via OSPF dynamic routing.

- D — EIGRP — learned via EIGRP dynamic routing.

Auto-added: When you configure an IP and run no shutdown, two routes auto-appear: Connected (C) and Local (L). If the interface is disabled, neither shows.

### Routing Table Components

📋 Exam ref: 3.1.a-g — Protocol code, prefix, mask, next hop, AD, metric, gateway of last resort

| Component | Description |
| --- | --- |
| Protocol code | Letter showing how route was learned (C, L, S, O, D) |
| Prefix / Network | Destination network address (e.g. 192.168.1.0) |
| Network mask | Subnet mask / prefix length (e.g. /24) |
| Next hop | IP address of the next router in the path |
| Admin Distance (AD) | Trustworthiness of the routing source. Lower = preferred. Static=1, OSPF=110, EIGRP=90 |
| Metric | Cost to reach destination. Lower = better. Varies by protocol. |
| Gateway of last resort | Default route (0.0.0.0/0) — used when no specific match found. |

## Route Selection

📋 Exam ref: 3.2 — How a router makes a forwarding decision

- Longest prefix match wins first — most specific route always used.

- If same prefix length → lowest Administrative Distance wins.

- If same AD → lowest metric wins.

## Default Gateway / Default Route

- Default route = 0.0.0.0/0 — matches ALL IP addresses. The least specific route possible.

- End hosts only need a default gateway — send everything outside my local network to the gateway.

## Static Routes

📋 Exam ref: 3.3 — Configure and verify static routing

- Network route — ip route <network> <mask> <next-hop>. Routes to a specific subnet.

- Default route — ip route 0.0.0.0 0.0.0.0 <next-hop>. Catches all unmatched destinations.

- Host route — ip route <host-IP> 255.255.255.255 <next-hop>. Routes to a single host (/32).

- Floating static — a static route with a higher AD than a dynamic route. Used as backup. Only activates if the primary route fails.

Two-way reachability: Each router needs a route TO the destination AND a return path. Without both, only one direction works.

- [1/0] in routing table = Administrative Distance / Metric.

  - Administrative Distance (AD) — how trustworthy is the source? Lower AD = more trusted. Static = 1.

  - Metric — how good is the path? Static routes metric = 0 (you typed it in manually).

  - Two routes to same destination: lowest AD wins first. If equal AD, lowest metric wins.

| Source | AD | Why? |
| --- | --- | --- |
| Directly Connected | 0 | Router is attached — certain |
| Static Route | 1 | Admin typed it — trusted |
| EIGRP | 90 | Cisco, fast convergence |
| OSPF | 110 | Open standard, common |
| RIP | 120 | Old, hop count only |

⌨  Key Commands for this section

| Command | Purpose | Description | Device |
| --- | --- | --- | --- |
| show ip route | View routing table | Displays all routes including connected, local, static, and dynamic. | Both |
| ip route <net> <mask> <next-hop> | Configure static route | Manually adds a route — router sends matching packets to the next-hop IP. | Router |
| ip route 0.0.0.0 0.0.0.0 <next-hop> | Configure default route | Gateway of last resort — all unmatched packets go to this next-hop. | Router |
| router ospf <process-id> | Enable OSPF | Starts OSPF configuration and assigns a process ID. | Router |
| network <IP> <wildcard> area <area-id> | Advertise network in OSPF | Declares networks participating in OSPF and assigns them to an area. | Router |
| router eigrp <AS-number> | Enable EIGRP | Starts EIGRP configuration and specifies the autonomous system number. | Router |
| router bgp <AS-number> | Enable BGP | Starts BGP configuration and specifies the autonomous system number. | Router |
| neighbor <IP> | Configure BGP neighbor | Establishes a BGP peer relationship with a specified IP address. | Router |
| show ip ospf neighbor | View OSPF neighbors | Lists OSPF neighbors and their states. | Router |
| show ip eigrp neighbors | View EIGRP neighbors | Lists EIGRP neighbors and their states. | Router |
| show ip bgp summary | View BGP summary | Provides a summary of BGP neighbors and their status. | Router |
| debug ip routing | Debug routing activity | Monitors routing protocol operations in real time. | Router |

## OSPF

📋 Exam ref: 3.4 — Configure and verify single area OSPFv2

⚠ GAP — 3.4: Single Area OSPFv2  (Add notes from video when studied)

Cover: neighbor adjacencies, DR/BDR election, router ID, point-to-point vs broadcast, full OSPF config.

## First Hop Redundancy Protocols

📋 Exam ref: 3.5 — First hop redundancy protocols

⚠ GAP — 3.5: First Hop Redundancy (HSRP, VRRP, GLBP)  (Add notes from video when studied)

Cover: purpose (avoid single gateway failure), HSRP active/standby router, virtual IP, preemption.

# Part 10 — Switch Operation & VLANs

📋 Exam ref: 2.0 — Network Access — 20% of exam

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| VLAN | Virtual Local Area Network | Logical grouping of network devices used to segment broadcast domains. |
| VTP | VLAN Trunking Protocol | Cisco protocol managing VLAN information across switches in a network. |
| DTP | Dynamic Trunking Protocol | Cisco proprietary protocol used to negotiate VLAN trunking between switches. |
| ISL | Inter-Switch Link | Cisco proprietary VLAN trunking protocol — mostly obsolete, replaced by 802.1q. |
| TPID | Tag Protocol Identifier | 16-bit 802.1q field (always 0x8100) identifying the frame as VLAN-tagged. |
| TCI | Tag Control Information | 16-bit 802.1q field containing PCP, DEI, and VID sub-fields. |
| PCP | Priority Code Point | 3-bit field in 802.1q tag used for Class of Service traffic prioritisation. |
| DEI | Drop Eligible Indicator | 1-bit 802.1q field marking frames that can be dropped during congestion. |
| VID | VLAN Identifier | 12-bit 802.1q field identifying which VLAN the frame belongs to (range 1–4094). |
| CoS | Class of Service | 3-bit field in 802.1q tags used to prioritise network traffic at Layer 2. |
| STP | Spanning Tree Protocol | Prevents Layer 2 loops by blocking redundant paths in a switched network. |
| CDP | Cisco Discovery Protocol | Cisco proprietary protocol that discovers directly connected Cisco devices. |

## Switch Interfaces

📋 Exam ref: 2.1 — Configure and verify VLANs

- show interface status columns: Port | Name | Status | VLAN | Duplex | Speed | Type

- Speed/duplex auto-negotiation — both sides advertise capabilities and negotiate the best match.

- If auto-negotiation disabled on connected device: switch defaults to slowest speed; half duplex (if 10/100 Mbps), full duplex (if 1000+ Mbps).

## VLANs

- VLAN — logically separates devices at Layer 2 into different broadcast domains.

- VLAN 1 — default VLAN, active on all ports.

- VLAN 1002–1005 — reserved, no longer used.

- Normal range: 1–1005  |  Extended range: 1006–4094

- Some older devices cannot support the extended VLAN range (1006–4094).

### Access vs Trunk Ports

📋 Exam ref: 2.1.a / 2.2.a — Access ports and trunk ports

- Access port — belongs to one VLAN. Connects to end devices. Untagged.

- Trunk port — carries multiple VLANs. Tagged using 802.1q. Connects switches or a router.

### 802.1q Trunking

📋 Exam ref: 2.2.b — 802.1Q

- ISL — Cisco proprietary, mostly obsolete.

- 802.1q (Dot1q) — industry standard. Adds a 4-byte (32-bit) tag between source MAC and Type/Length field.

| Field | Size | Description |
| --- | --- | --- |
| TPID (Tag Protocol Identifier) | 16 bits | Always 0x8100 — identifies 802.1q tagged frame. |
| PCP (Priority Code Point) | 3 bits | Class of Service (CoS) — prioritises traffic in congested networks. |
| DEI (Drop Eligible Indicator) | 1 bit | Marks frames that can be dropped during congestion. |
| VID (VLAN Identifier) | 12 bits | Identifies the VLAN the frame belongs to (range 1–4094). |

### Native VLAN

📋 Exam ref: 2.2.c — Native VLAN

- Native VLAN — the VLAN sent UNTAGGED on a trunk port. Default is VLAN 1.

- VLAN 1 is the default native VLAN on all Cisco trunk ports.

- Untagged frames arriving on a trunk are assumed to belong to the native VLAN.

- Both ends of a trunk MUST have the same native VLAN — mismatched native VLANs cause misrouted frames.

- switchport trunk native vlan <vlan-id>Native VLAN configuration:

- show interfaces trunkVerify native VLAN:

Security — VLAN Hopping: An attacker can craft double-tagged 802.1q frames to jump from one VLAN to another. This only works if the attacker is on the native VLAN. Fix: change the native VLAN to an unused VLAN (not VLAN 1, not any user VLAN).

### Inter-VLAN Routing

📋 Exam ref: 2.1.c — InterVLAN connectivity

- Router-on-a-stick — one physical router port split into sub-interfaces (one per VLAN). Trunk port on the switch side.

- SVI (Switched Virtual Interface) — virtual interface on a Layer 3 switch, one per VLAN. Faster than router-on-a-stick.

- Sub-interface: interface g0/0.10 → encapsulation dot1q 10 → ip address x.x.x.x mask

- SVI: interface vlan 10 → ip address x.x.x.x mask → no shutdown

## Layer 2 Discovery Protocols

📋 Exam ref: 2.3 — Layer 2 discovery protocols — CDP and LLDP

- CDP (Cisco Discovery Protocol) — Cisco proprietary. Discovers directly connected Cisco devices. Shares hostname, IP, model, interface.

- LLDP (Link Layer Discovery Protocol) — vendor-neutral equivalent of CDP. IEEE 802.1AB standard.

## Spanning Tree Protocol (STP)

📋 Exam ref: 2.5 — Interpret basic operations of Rapid PVST+

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| BPDU | Bridge Protocol Data Unit | Hello messages sent between STP switches every 2 seconds to share topology information. |
| STP | Spanning Tree Protocol | Layer 2 protocol (IEEE 802.1D) that prevents loops by blocking redundant ports. |

### Why STP Exists

- Networks need redundant links for 24/7 uptime — if one link fails, another takes over.

- Redundant Layer 2 links without STP create broadcast storms — traffic loops endlessly, crashing the network.

- STP fixes this — blocks redundant ports to prevent loops, but keeps them ready as backup.

- Classic STP = IEEE 802.1D. Runs on all switches by default across all vendors.

### BPDUs (Bridge Protocol Data Units)

- Switches send Hello BPDUs out all interfaces every 2 seconds.

- If a switch receives a Hello BPDU — another switch is on that port.

- Routers and PCs do NOT use STP and do not send BPDUs.

### STP Port States

| State | Type | Receives BPDUs | Learns MACs | Forwards Traffic |
| --- | --- | --- | --- | --- |
| Blocking | Stable | Yes | No | No |
| Listening | Transitional (15s) | Yes | No | No |
| Learning | Transitional (15s) | Yes | Yes | No |
| Forwarding | Stable | Yes | Yes | Yes |
| Disabled | Admin down | No | No | No |

- Blocking and Forwarding are the two stable states.

- Listening and Learning are transitional — passed through when activating or recovering a port.

- Non-designated ports stay in Blocking. Root and designated ports stay in Forwarding.

### STP Port Roles

- Root port — best path back to the root bridge. One per non-root switch. Always Forwarding.

- Designated port — forwarding port on each segment. One per segment. Always Forwarding.

- Non-designated port — blocking port. Prevents loops.

### STP Timers

| Timer | Default | What it controls |
| --- | --- | --- |
| Hello | 2 sec | How often the root bridge sends Hello BPDUs. |
| Forward Delay | 15 sec | How long a port stays in Listening AND Learning (15s each = 30s total). |
| Max Age | 20 sec | How long a switch waits without a BPDU before reconverging. |

Convergence: Blocking → Listening (15s) → Learning (15s) → Forwarding = 50 seconds total.

- Max Age — BPDU received before 0: timer resets, no changes.

- Max Age — no BPDU, timer hits 0: switch re-evaluates root bridge, root ports, designated and non-designated ports from scratch.

### Root Bridge Election

- What a root bridge is: the single reference point the whole STP topology in a VLAN is built around. Every other switch calculates its lowest-cost path back to the root bridge, and that calculation decides each port’s role — Root Port, Designated Port, or Blocking. The root bridge originates the Hello BPDUs every 2 seconds; every non-root switch just forwards them on rather than generating its own.

- The switch with the lowest Bridge ID becomes the root bridge.

- Bridge ID = Bridge Priority (default 32768) + MAC address. Lowest wins.

- Manually configure root bridge by lowering bridge priority.

- Secondary root bridge can be configured as a backup.

### PortFast

- PortFast — skips Listening and Learning, goes straight to Forwarding.

- Only use on access ports to end devices. NEVER on ports connecting to other switches.

- Edge — PortFast for end device ports (standard use).

- Network — bridge assurance (not a CCNA topic).

- Can also be used on trunk ports for: virtual servers with VMs in different VLANs, or router-on-a-stick.

### STP Toolkit — BPDU Guard and BPDU Filter

- A PortFast-enabled port still sends BPDUs every 2 seconds and, unlike a normal STP port, isn't expecting to receive any back — end hosts don't run STP. If a switch gets connected to that port by mistake, the port starts receiving BPDUs and reverts to acting like a regular STP port. That can affect the STP topology. BPDU Guard and BPDU Filter are the two protection features that address this, and they solve different halves of the problem.

- BPDU Guard — protects the network from an unauthorised switch being connected to a port meant for an end host. The port keeps sending its own BPDUs as normal, but if it receives one, it goes into error-disabled (err-disabled) state — in effect, the port is shut down.

- BPDU Filter — stops a port from sending BPDUs in the first place (and ignores any it receives). Sending BPDUs to end-host ports costs a small amount of bandwidth/CPU and leaks STP topology information — undesirable if you're securing user-facing ports. Unlike BPDU Guard, BPDU Filter does not disable the port on receipt of a BPDU.

- Both are usually enabled alongside PortFast on host-facing ports, but each can be configured independently.

- Configuring BPDU Guard: per port (interface config mode) — spanning-tree bpduguard enable. Default, all PortFast ports (global config mode) — spanning-tree portfast bpduguard default, disable on a specific port with spanning-tree bpduguard disable.

- Configuring BPDU Filter: per port (interface config mode) — spanning-tree bpdufilter enable, which stops the port sending BPDUs and makes it ignore any received (in effect disables STP on the port, so use with caution). Default, all PortFast ports (global config mode) — spanning-tree portfast [edge] bpdufilter default — if a BPDU is received on one of these ports, PortFast and BPDU Filter both switch off and the port reverts to operating as a normal STP port.

- Recommendation: enable PortFast and BPDU Guard however suits (per-port or default) — enable BPDU Filter by default only, unless there's a specific reason to enable it per port.

- When both are enabled on the same port and a BPDU arrives:

| Where BPDU Filter was enabled | What happens |
| --- | --- |
| Globally (default) | BPDU Filter disables the port and BPDU Guard is triggered (err-disabled). |
| Per-interface | The BPDU is simply ignored — BPDU Guard is not triggered. |

### Root Guard and Loop Guard

- Two further STP toolkit features, introduced briefly here — full command coverage to follow when studied in depth.

- Root Guard — stops a port from ever becoming a root port. If the port receives a superior BPDU (one that would normally win root-bridge election), Root Guard puts it into a root-inconsistent state instead of letting the topology change. Enforces the current root bridge against a misconfigured or rogue switch claiming to be a better root.

- Loop Guard — protects the network from loops caused by a unidirectional link — a link where a switch can send BPDUs but has stopped receiving them (or vice versa), typically from a software bug, or a hardware fault causing one direction of a link to fail while the other keeps working.

### Why it happens — Unidirectional Links

- A unidirectional link is a link where data only flows one way — e.g. SW1 can send frames to SW2, but SW2 can't send frames back to SW1. Almost always a Layer 1 issue: a damaged cable, or a faulty connector/transceiver.

- More common on fibre than copper UTP. A fibre connection is really two separate strands (one Tx, one Rx) bundled together as “one cable” — SW1's Tx strand feeds SW2's Rx port, and SW2's Tx strand feeds SW1's Rx port. Both strands need to be intact for the interface to come up/up. If only one strand is damaged, the link can end up passing traffic in one direction while the other direction silently fails — and if the devices don't detect that specific failure mode, the result is a unidirectional link.

- Loop Guard doesn't prevent unidirectional links themselves (that's a Layer 1 problem) — it protects STP from the loop that a unidirectional link can otherwise cause.

### How Loop Guard Reacts

- Normally, if a non-designated port stops receiving BPDUs, its Max Age timer counts down and the port transitions to designated/forwarding — exactly the scenario Loop Guard exists to stop. On a Loop Guard-enabled port, when Max Age hits 0 without a BPDU, the port does not become designated and does not start forwarding. Instead it enters the Broken (loop-inconsistent) state — the same “Broken” family as the root-inconsistent state Root Guard triggers. In both cases the port stays up/up physically, but STP blocks it.

- Loop Guard should be enabled on Root and Non-Designated ports — the ports that are supposed to be receiving BPDUs.

### Loop Guard and Root Guard Are Mutually Exclusive

- They can't both be enabled on the same port at the same time. Root Guard stops a Designated port from becoming a Root Port; Loop Guard stops a Non-Designated or Root Port from becoming a Designated port — opposite ends of the same problem, which is why Cisco doesn't allow both at once.

- If Loop Guard is configured per-port (spanning-tree guard loop) and Root Guard is then configured on that same port (spanning-tree guard root), Loop Guard gets switched off — and vice versa.

- If Loop Guard is enabled globally by default (spanning-tree loopguard default) and Root Guard is then configured on a specific port, Loop Guard is disabled on that port. The more specific configuration (interface) beats the more general one (global).

- Use spanning-tree guard none on an interface to remove either Root Guard or Loop Guard from that port.

- Verification: a violation logs %SPANTREE-2-LOOPGUARD_BLOCK: Loop guard blocking port <interface> on <VLAN>, and show spanning-tree shows the port as BKN (broken) with a *LOOP_Inc flag against it.

- Confirmed: the earlier note above about a “Root Filter” matches Loop Guard exactly, as detailed above — not a separate feature.

### ErrDisable and ErrDisable Recovery

- ErrDisable is a Cisco feature that shuts a port down administratively when it detects certain violations. On the CCNA, the relevant triggers are: BPDU Guard violation, port security violation, power policing (PoE) violation, and DAI (Dynamic ARP Inspection) violation.

- Re-enabling an err-disabled port without fixing the underlying issue just puts it straight back into err-disabled — fix the cause first. Two ways to recover a port:

- Manual — shutdown then no shutdown on the interface.

- Automatic — ErrDisable Recovery — disabled by default. When enabled, automatically re-enables err-disabled ports after a timer (default 300 seconds / 5 minutes).

### Rapid Spanning Tree Protocol (RSTP)

- RSTP is not a timer-based spanning tree algorithm like 802.1D. It offers an improvement over the 30+ seconds classic STP takes to move a link to forwarding. The heart of the protocol is a new bridge-to-bridge handshake mechanism, which allows ports to move directly to forwarding.

- Similarities between STP and RSTP: RSTP serves the same purpose as STP — blocking specific ports to prevent Layer 2 loops. It elects a root bridge, root ports, and designated ports with the same rules as STP.

### RSTP Port Costs

- RSTP expanded the cost scale to handle faster link speeds — classic STP's scale can't distinguish above 10 Gbps.

| Speed | STP Cost | RSTP Cost |
| --- | --- | --- |
| 10 Mbps | 100 | 2,000,000 |
| 100 Mbps | 19 | 200,000 |
| 1 Gbps | 4 | 20,000 |
| 10 Gbps | 2 | 2,000 |
| 100 Gbps | — | 200 |
| 1 Tbps | — | 20 |

### RSTP Port States

- RSTP simplifies classic STP's five port states down to three: Blocking, Listening, and Disabled are merged into a single Discarding state.

| RSTP Port State | Send/Receive BPDUs | Frame forwarding | MAC learning | Stable/Transitional |
| --- | --- | --- | --- | --- |
| Discarding | No / Yes | No | No | Stable |
| Learning | Yes / Yes | No | Yes | Transitional |
| Forwarding | Yes / Yes | Yes | Yes | Stable |

  - If a port is administratively disabled (shutdown command) = Discarding state.

  - If a port is enabled but blocking traffic to prevent Layer 2 loops = Discarding state.

### RSTP Port Roles

- Root port — role unchanged in RSTP. The port that is closest to the root bridge becomes the root port for the switch. The root bridge is the only switch that doesn't have a root port.

- Designated port — role unchanged in RSTP. The port on a segment (collision domain) that sends the best BPDU is that segment's designated port — only one per segment.

- Non-designated port — split into two separate roles in RSTP: Alternate and Backup.

  - Alternate port — a discarding port that receives a superior BPDU from another switch. Same as blocking ports in classic STP. Functions as a backup to the root port — if the root port fails, the switch can immediately move its best alternate port to forwarding.

  - Note: this immediate move to forwarding functions like the classic STP optional feature UplinkFast. Because it is built into RSTP, you do not need to activate UplinkFast when using RSTP/Rapid PVST+.

  - Backup port — a discarding port that receives a superior BPDU from another interface on the same switch. This only happens when two interfaces are connected to the same collision domain (via a hub). Hubs aren't used in modern networks, so you will probably never encounter one. Functions as a backup for a designated port — the interface with the lowest port ID is selected as the designated port, and the other becomes the backup port.

- BackboneFast — another classic STP optional feature built into RSTP (no configuration needed). Allows a switch to immediately expire the Max Age timer on its interface and rapidly forward superior BPDUs when an indirect link failure is detected.

### RSTP BPDU Differences

- All switches originate BPDUs. In classic STP, only the root bridge originated BPDUs, and other switches just forwarded the ones they received. In Rapid STP, ALL switches originate and send their own BPDUs from their designated ports, every hello time (2 seconds).

- Faster ageing. In classic STP, a switch waits 10 hello intervals (20 seconds) before considering a neighbour lost. In Rapid STP, a neighbour is considered lost after 3 missed BPDUs (6 seconds), and the switch then flushes all MAC addresses learned on that interface.

### RSTP Link Types

- Edge — a port connected to an end host. Moves directly to forwarding, without negotiation — because there's no risk of creating a loop. Functions like a classic STP port with PortFast enabled. Configured with: spanning-tree portfast.

- Point-to-Point — a direct connection between two switches, functioning in full duplex mode. Doesn't need to be configured manually (it's detected automatically). Command: spanning-tree link-type point-to-point.

- Shared — a connection to a hub, which must operate in half duplex mode. Also detected automatically. Command: spanning-tree link-type shared.

### Spanning Tree Versions — IEEE vs Cisco

| IEEE standard | Cisco version |
| --- | --- |
| Spanning Tree Protocol (802.1D) — the original STP. All VLANs share one STP instance, so it cannot load balance. | Per-VLAN Spanning Tree (PVST+) — Cisco's upgrade to 802.1D. Each VLAN has its own STP instance, so it can load balance by blocking different ports in each VLAN. |
| Rapid Spanning Tree Protocol (802.1w) — much faster at converging/adapting to network changes than 802.1D. All VLANs share one instance, so load balancing is not supported. | Rapid Per-VLAN Spanning Tree Plus (Rapid PVST+) — Cisco's upgrade to 802.1w. Each VLAN has its own STP instance, so it can load balance by blocking different ports in each VLAN. |
| Multiple Spanning Tree Protocol (802.1s) — uses modified RSTP mechanics. Can group multiple VLANs into different instances (e.g. VLANs 1–5 in instance 1, VLANs 6–10 in instance 2) to load balance. | — |

⌨  Key Commands for this section

| Command | Purpose | Description | Device |
| --- | --- | --- | --- |
| spanning-tree vlan <id> root primary | Set root bridge | Lowers bridge priority to make this switch the root bridge for the VLAN. | Switch |
| spanning-tree vlan <id> root secondary | Set secondary root bridge | Configures this switch as backup root bridge if the primary fails. | Switch |
| spanning-tree portfast | PortFast on a port | Interface mode. Port skips Listening/Learning, goes straight to Forwarding. | Switch |
| spanning-tree portfast default | PortFast on all access ports | Global mode. Enables PortFast on all access ports automatically. | Switch |
| show spanning-tree | View STP topology | Shows root bridge, port roles, and port states for all VLANs. | Switch |
| show spanning-tree int <int> detail | STP detail for a port | Detailed STP info for a specific interface including timers and port role. | Switch |
| spanning-tree bpduguard enable | BPDU Guard on a port | Interface mode. Err-disables the port if it receives a BPDU. | Switch |
| spanning-tree portfast bpduguard default | BPDU Guard on all PortFast ports | Global mode. Enables BPDU Guard by default wherever PortFast is enabled. | Switch |
| spanning-tree bpduguard disable | Disable BPDU Guard on a port | Overrides the global default on a specific interface. | Switch |
| spanning-tree bpdufilter enable | BPDU Filter on a port | Interface mode. Stops the port sending/processing BPDUs. Use with caution. | Switch |
| spanning-tree portfast [edge] bpdufilter default | BPDU Filter on all PortFast ports | Global mode. Enables BPDU Filter by default wherever PortFast is enabled. | Switch |
| spanning-tree bpdufilter disable | Disable BPDU Filter on a port | Overrides the global default on a specific interface. | Switch |
| errdisable recovery cause bpduguard | Enable auto-recovery for BPDU Guard | Turns on ErrDisable Recovery for ports disabled by a BPDU Guard violation. | Switch |
| errdisable recovery interval <seconds> | Set recovery timer | Changes how long a port stays err-disabled before auto-recovery (default 300s). | Switch |
| show errdisable recovery | View recovery settings | Shows which causes have auto-recovery enabled and the current timer. | Switch |
| spanning-tree guard loop | Enable Loop Guard on a port | Interface mode. Puts the port into loop-inconsistent state if it stops receiving BPDUs unexpectedly. | Switch |
| spanning-tree loopguard default | Loop Guard on all ports | Global mode. Enables Loop Guard by default on every port. | Switch |
| spanning-tree guard none | Disable Root Guard / Loop Guard on a port | Overrides Root Guard or Loop Guard on a specific interface. | Switch |
| spanning-tree link-type point-to-point | Set RSTP link type: point-to-point | Full-duplex link between two switches. Normally auto-detected — manual config rarely needed. | Switch |
| spanning-tree link-type shared | Set RSTP link type: shared | Half-duplex link to a hub. Normally auto-detected — manual config rarely needed. | Switch |

## EtherChannel

📋 Exam ref: 2.4 — Configure and verify EtherChannel (LACP)

- EtherChannel groups multiple physical interfaces together so they act as a single logical interface (also called a Port Channel or LAG — Link Aggregation Group).

- Why it exists: if two switches are connected by multiple separate links, STP will block all but one of them to prevent a Layer 2 loop — if every link were left forwarding, a loop would form between the switches and cause a broadcast storm. The blocked links sit unused unless the active link fails, at which point one of them takes over.

- EtherChannel's fix: STP treats the whole bundle as a single interface, so none of the member links get blocked — all of them forward traffic simultaneously, and the switch load-balances across them.

- Oversubscription: when the combined bandwidth of the interfaces connecting end hosts to an access switch (ASW) is greater than the bandwidth of that switch's uplink to the distribution switch (DSW). Some oversubscription is normal and acceptable, but too much causes congestion — EtherChannel is one way to raise the uplink capacity and reduce it.

### Load Balancing — Flows

- A flow is a communication between two nodes on the network. EtherChannel load-balances per flow, not per frame.

- All frames in the same flow use the same physical interface. If frames from the same flow were spread across different physical interfaces, some could arrive at the destination out of order, which causes problems for the receiving end. An algorithm decides which flow uses which physical interface.

- The inputs to that algorithm can be changed. Options: source MAC, destination MAC, source and destination MAC, source IP, destination IP, or source and destination IP.

### EtherChannel Configuration Methods

- PAgP (Port Aggregation Protocol) — Cisco-proprietary. Dynamically negotiates the creation/maintenance of the EtherChannel, the same way DTP does for trunks. Modes: auto or desirable.

- LACP (Link Aggregation Control Protocol) — industry standard (IEEE 802.3ad). Dynamically negotiates the creation/maintenance of the EtherChannel. The preferred method. Modes: active or passive.

- Static EtherChannel — no protocol is used to negotiate whether the EtherChannel should form — interfaces are just statically configured to be in one. Mode: on.

### Negotiation Compatibility

- PAgP: auto + auto = no EtherChannel

  - desirable + auto = EtherChannel

  - desirable + desirable = EtherChannel

- LACP: passive + passive = no EtherChannel

  - active + passive = EtherChannel

  - active + active = EtherChannel

- On mode only works with on mode — on + desirable or on + active will not work, since “on” doesn't negotiate at all.

### Interfaces, Channel Groups, and the Port Channel

- Up to 8 active interfaces per EtherChannel. LACP technically allows up to 16 members — 8 active, with the other 8 in standby, ready to take over if an active interface fails.

- channel-group is the command used to create the EtherChannel on member interfaces; the resulting logical interface itself is called a port-channel.

- The channel-group number must match across the member interfaces on the same switch. It does NOT have to match the channel-group number used on the other switch — channel-group 1 on SW1 can form an EtherChannel with channel-group 2 on SW2.

- After the EtherChannel is formed (in any mode), you configure the port-channel interface itself — e.g. interface port-channel 1 — the same way you'd configure any other switch interface (trunk, access, etc.). The member physical interfaces then inherit that configuration.

- Member interfaces must match on: duplex, speed, switchport mode (access/trunk), and allowed VLANs/native VLAN (for trunk interfaces). An interface whose configuration doesn't match the others is excluded from the EtherChannel rather than breaking it.

⌨  Key Commands for this section

| Command | Purpose | Description | Device |
| --- | --- | --- | --- |
| show interface status | Switch port status summary | Shows port, name, status, VLAN, duplex, speed, and type in compact form. | Switch |
| show vlan brief | VLAN summary | Lists all VLANs and which access ports are assigned to each. | Switch |
| vlan <vlan-id> | Create/enter a VLAN | Creates a new VLAN with the specified ID and enters VLAN config mode. | Switch |
| name <vlan-name> | Name a VLAN | Assigns a descriptive name to the VLAN (e.g. name SALES). | Switch |
| interface range <type> <range> | Configure multiple interfaces | Applies the same config to a range of interfaces at once (e.g. int range g0/1 - 12). | Both |
| switchport mode access | Set port to access mode | Configures the port to connect to a single end device (one VLAN only). | Switch |
| switchport access vlan <id> | Assign port to a VLAN | Places the access port into the specified VLAN. | Switch |
| switchport mode trunk | Set port to trunk mode | Allows the port to carry traffic for multiple VLANs. | Switch |
| show mac address-table | View MAC address table | Lists MAC addresses learned by the switch, with VLAN, type, and port. | Switch |
| show cdp neighbors | View CDP neighbor list | Lists directly connected Cisco devices and their interfaces. | Both |
| speed <value> | Set interface speed | Manually sets the speed (10/100/1000 Mbps) instead of auto-negotiating. | Both |
| duplex <mode> | Set duplex mode | Manually sets full or half duplex instead of auto-negotiating. | Both |
| ip default-gateway <IP> | Set default gateway on switch | Configures the IP address of the gateway for out-of-subnet traffic (switches without routing only). | Switch |
| channel-group <number> mode {desirable¦auto¦active¦passive¦on} | Add an interface to an EtherChannel | Interface mode. Creates/joins the EtherChannel using PAgP (desirable/auto), LACP (active/passive), or statically (on). | Switch |
| interface port-channel <number> | Configure the EtherChannel interface | Configures the logical port-channel interface itself (trunk/access, VLANs, etc.) — member interfaces inherit it. | Switch |
| channel-protocol {pagp¦lacp} | Manually set the negotiation protocol | Sets which protocol the member interfaces should use, without setting a channel-group mode yet. | Switch |
| port-channel load-balance <mode> | Set the load-balancing method | Global config. Changes the algorithm input (e.g. src-dst-mac, src-dst-ip) used to assign flows to physical interfaces. | Switch |
| show etherchannel summary | View EtherChannel status | Summarises all EtherChannels on the switch, their member interfaces, and protocol. | Switch |
| show etherchannel port-channel | View port-channel interfaces | Displays information about the virtual port-channel interfaces on the switch. | Switch |
| show etherchannel load-balance | View load-balancing settings | Displays the current load-balancing method configured on the switch. | Switch |

## Wireless

📋 Exam ref: 2.6 / 2.7 / 1.11 — Wireless architectures, WLAN components, wireless principles

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| SSID | Service Set Identifier | Human-readable name shared by all devices in a service set; need not be unique. |
| BSS | Basic Service Set | Clients connect through one AP, not directly to each other. |
| BSSID | Basic Service Set Identifier | Uniquely identifies an AP; it is the MAC address of the AP's radio. |
| BSA | Basic Service Area | Physical area around an AP where its signal is usable. |
| IBSS | Independent Basic Service Set | Ad-hoc network where devices connect directly with no AP. |
| ESS | Extended Service Set | Multiple BSSs joined by a wired network to extend coverage. |
| MBSS | Mesh Basic Service Set | Mesh of APs using a wireless backhaul where cabling every AP is impractical. |
| DS | Distribution System | The upstream wired network that a WLAN connects into. |
| AP | Access Point | Device that provides wireless connectivity to clients. |
| WLC | Wireless LAN Controller | Centrally manages multiple APs. |
| RAP | Root Access Point | Mesh AP that is connected to the wired network. |
| MAP | Mesh Access Point | Mesh AP that reaches the wired network via the backhaul. |
| WGB | Workgroup Bridge | AP acting as a wireless client to connect wired devices (Cisco version bridges many). |
| UWGB | Universal Workgroup Bridge | 802.11-standard WGB that bridges a single wired device. |
| CSMA/CA | Carrier Sense Multiple Access / Collision Avoidance | Half-duplex access method that avoids collisions on wireless. |
| RF | Radio Frequency | Electromagnetic waves (~30 Hz-300 GHz) used to carry wireless signals. |
| WLAN | Wireless LAN | Local network using IEEE 802.11 wireless instead of cabling. |
| CAPWAP | Control and Provisioning of Wireless APs | WLC–AP protocol; control tunnel UDP 5246, data tunnel UDP 5247. |
| LWAPP | Lightweight Access Point Protocol | Older protocol that CAPWAP is based on. |
| DTLS | Datagram Transport Layer Security | Optional encryption for the CAPWAP data tunnel. |
| RTS | Request To Send | Control frame used to reserve the medium before sending. |
| CTS | Clear To Send | Control frame granting the medium after an RTS. |
| ACK | Acknowledgement | Control frame confirming a frame was received. |
| HT | High Throughput | 802.11n high-throughput operation. |
| DA | Destination Address | 802.11 address: final recipient. |
| SA | Source Address | 802.11 address: original sender. |
| RA | Receiver Address | 802.11 address: immediate recipient. |
| TA | Transmitter Address | 802.11 address: immediate sender. |
| PoE | Power over Ethernet | Power + data to an AP over one Ethernet cable. |
| LAG | Link Aggregation Group | EtherChannel uplink; static-only on AireOS WLCs. |
| PAgP | Port Aggregation Protocol | Cisco EtherChannel negotiation (not on AireOS WLCs). |
| LACP | Link Aggregation Control Protocol | Standard EtherChannel negotiation (802.3ad). |
| PSK | Pre-Shared Key | Shared password for WPA/WPA2 personal. |
| WPA2 | Wi-Fi Protected Access 2 | Current wireless security standard (AES-CCMP). |
| HA | High Availability | Active/standby WLC pair via redundancy ports. |
| OOB | Out Of Band | Management over a separate path (WLC service port). |
| TFTP | Trivial File Transfer Protocol | Simple UDP transfer, e.g. to load a WLC config. |
| SVI | Switched Virtual Interface | Logical L3 VLAN interface on a switch (WLC interface analogy). |

802.11 & Wi-Fi standards

- WLAN standards are defined by IEEE 802.11.

- "Wi-Fi" is a trademark of the Wi-Fi Alliance, which is closely tied to the IEEE.

- The Wi-Fi Alliance tests and certifies equipment for 802.11 standards compliance and interoperability with other devices.

Shared medium, half-duplex & CSMA/CA

- Like devices on an Ethernet hub, all wireless devices in range receive all frames, so privacy of data within the WLAN is a greater concern.

- Wireless links are half-duplex, so CSMA/CA (Carrier Sense Multiple Access with Collision Avoidance) is used to coordinate access.

- Wired Ethernet uses CSMA/CD to detect and recover from collisions; wireless cannot detect collisions, so CSMA/CA avoids them instead.

- With CSMA/CA a device waits for others to stop transmitting first: assemble frame -> is the channel free? -> if yes, transmit; if no, wait a period and check again.

Signal coverage & propagation

- Wireless communications are regulated by various international and national bodies, and coverage area / signal range must be planned.

- Absorption — signal passes through a material and is converted to heat, weakening it.

- Reflection — signal bounces off a material such as metal (why reception is poor in a metal elevator).

- Refraction — signal bends when entering a medium where it travels at a different speed.

- Diffraction — signal bends around an obstacle, which can create blind spots behind it.

- Scattering — signal scatters in all directions off dust, smog or uneven surfaces.

- Interference — other devices on the same channel (e.g. a neighbour's WLAN) degrade the signal.

Radio frequency (RF) basics

- To transmit, an alternating current is applied to an antenna, creating electromagnetic fields that propagate outward as waves.

- Amplitude — the maximum strength of the electric and magnetic fields.

- Frequency — the number of cycles per unit of time, measured in hertz (Hz = cycles per second): kHz = 10^3, MHz = 10^6, GHz = 10^9, THz = 10^12 cycles per second.

- Period — the time for one cycle; period = 1 / frequency (e.g. 4 Hz gives a period of 0.25 s).

- Visible light is roughly 400-790 THz; the radio-frequency range used for wireless is about 30 Hz to 300 GHz.

Wi-Fi bands & channels

- 2.4 GHz band — actual range 2.400-2.4835 GHz. Longer reach in open space and better penetration of walls, but more crowded, so interference is a bigger problem.

- 5 GHz band — actual range 5.150-5.825 GHz, split into four sub-bands: 5.150-5.250, 5.250-5.350, 5.470-5.725 and 5.725-5.825 GHz. More channels and less congestion. Modern devices support both bands.

- Wi-Fi 6 (802.11ax) — expanded the spectrum to add a 6 GHz band; operation in that 6 GHz band is branded Wi-Fi 6E.

- Wi-Fi 7 (802.11be) — the newest standard; builds on 6 GHz with wider 320 MHz channels and higher throughput. Beyond CCNA scope, but useful context.

- Each band is divided into channels, and devices transmit/receive on one or more of them.

- 2.4 GHz channels are ~22 MHz wide but spaced only 5 MHz apart, so neighbouring channels overlap (exact set varies by country).

- Adjacent APs must not use overlapping channels; in 2.4 GHz use channels 1, 6 and 11, which do not overlap.

- 5 GHz channels are non-overlapping, so avoiding interference between adjacent APs is much easier.

- Using 1 / 6 / 11 you can lay APs out in a honeycomb pattern for full coverage with no co-channel interference.

📌  2.4 GHz channels (22 MHz wide, 5 MHz spacing)

| Channel | Centre freq (GHz) | Notes |
| --- | --- | --- |
| 1 | 2.412 | Recommended – non-overlapping |
| 2 | 2.417 | Overlaps 1 & 6 |
| 3 | 2.422 | Overlaps |
| 4 | 2.427 | Overlaps |
| 5 | 2.432 | Overlaps |
| 6 | 2.437 | Recommended – non-overlapping |
| 7 | 2.442 | Overlaps |
| 8 | 2.447 | Overlaps |
| 9 | 2.452 | Overlaps |
| 10 | 2.457 | Overlaps |
| 11 | 2.462 | Recommended – non-overlapping |
| 12 | 2.467 | Not usable in some countries |
| 13 | 2.472 | Not usable in some countries |
| 14 | 2.484 | Japan only (802.11b) |

Service sets (IBSS / BSS / ESS / MBSS)

- 802.11 defines Service Sets — groups of wireless devices. All devices in a service set share the same SSID (Service Set Identifier), a human-readable name that need not be unique.

- IBSS (Independent BSS) — two or more devices connect directly with no AP (ad-hoc); used for limited purposes such as file transfer and not scalable.

- BSS (Basic Service Set) — clients connect via an AP rather than directly to each other.

- BSSID uniquely identifies the AP and is the MAC address of the AP's radio; other APs may reuse the SSID but not the BSSID.

- Devices request to associate with the BSS; once associated they are called clients or stations.

- BSA (Basic Service Area) is the physical area around the AP where the signal is usable.

- ESS (Extended Service Set) — multiple BSSs joined by a wired network to extend the WLAN beyond one AP. Each BSS uses the same SSID, a unique BSSID, and a different channel to avoid interference.

- Clients move between APs without reconnecting = roaming; for seamless roaming, BSAs should overlap by about 15-20%.

- MBSS (Mesh BSS) — used where running Ethernet to every AP is difficult. Mesh APs use two radios: one for the client BSS and one for a wireless backhaul that bridges traffic between APs.

- The AP wired to the network is the RAP (Root AP); the others are MAPs (Mesh APs). A protocol picks the best path through the mesh, similar to a dynamic routing protocol.

Distribution System (DS)

- Most WLANs are not standalone; they connect wireless clients to the wired infrastructure. In 802.11 the upstream wired network is the Distribution System (DS).

- Each BSS or ESS maps to a VLAN on the wired network.

- One AP can provide multiple WLANs, each with its own SSID mapped to a separate VLAN and carried to the switch over a trunk (the AP connects to a switch trunk port).

- Each WLAN uses a unique BSSID, usually by incrementing the last digit of the AP radio's BSSID by one.

Additional AP operating modes

- Repeater — extends the range of a BSS by retransmitting the AP's signal. A single-radio repeater must use the same channel as the AP, which roughly halves throughput; a dual-radio repeater can receive on one channel and retransmit on another.

- Workgroup Bridge (WGB) — operates as a wireless client of another AP to connect wired devices. UWGB (Universal WGB) is the 802.11-standard version and bridges a single wired device; the Cisco-proprietary WGB bridges multiple wired devices.

- Outdoor bridge — connects networks over long distances with no cable, using directional antennas that focus most of the signal power in one direction. Can be point-to-point or point-to-multipoint (several sites to one central site).

802.11 frame format

- 802.11 frames have a different format from 802.3 Ethernet frames, and depending on the 802.11 version and message type some fields may be absent (e.g. not all messages use all four address fields).

- Frame Control — carries information such as the message type and subtype.

- Duration/ID — depending on message type, the time (in microseconds) the channel is reserved for the frame, or an identifier for the association.

- Addresses (up to 4) — which addresses appear and in what order depends on the message type: Destination (DA, final recipient), Source (SA, original sender), Receiver (RA, immediate recipient) and Transmitter (TA, immediate sender).

- Sequence Control — reassembles fragments and removes duplicate frames.

- QoS Control — used by QoS to prioritise certain traffic.

- HT (High Throughput) Control — added in 802.11n (also called High Throughput / HT Wi-Fi) to enable high-throughput operation.

- FCS (Frame Check Sequence) — used to check for errors.

802.11 association process

- APs bridge traffic between wireless stations and other devices; a station must be associated with the AP before it can send traffic through it.

- There are three connection states: (1) not authenticated, not associated; (2) authenticated, not associated; (3) authenticated, associated — only in state 3 can the station pass traffic.

- Sequence: station sends a probe request and the AP replies with a probe response (state 1) → station sends an auth request and the AP replies with an auth response (state 2) → station sends an association request and the AP replies with an association response (state 3).

- Active scanning — the station sends probe requests and listens for probe responses from APs.

- Passive scanning — the station listens for beacon messages, which APs send periodically to advertise the BSS.

📌  802.11 message types

| Type | Purpose / examples |
| --- | --- |
| Management | Manage the BSS — beacon, probe request/response, authentication, association request/response. |
| Control | Control access to the medium (RF) and assist delivery of management and data frames — RTS (Request To Send), CTS (Clear To Send), ACK. |
| Data | Carry the actual data packets. |

AP deployment models

- There are three main AP deployment methods: autonomous, lightweight (Split-MAC with a WLC) and cloud-based.

Autonomous APs

- Self-contained APs that need no WLC and are configured individually (via console/CLI, Telnet/SSH, or HTTP/HTTPS GUI); each needs an IP address for remote management.

- RF parameters (transmit power, channel, etc.) are set manually, and security and QoS policies are configured separately on every AP — there is no central monitoring or management.

- An autonomous AP connects to the wired network on a trunk link (even with a single SSID) because management sits on a separate management VLAN.

- Client data has a very direct path to the wired network or to other clients on the same AP.

- Each VLAN must stretch across the whole network, which is considered bad practice: large broadcast domains, spanning tree disables links, and adding/removing VLANs is labour-intensive.

- Workable in small networks but not viable in medium/large ones (which can have thousands of APs). Autonomous APs can also run the repeater, outdoor-bridge and workgroup-bridge modes described earlier.

Lightweight APs & Split-MAC architecture

- An AP's functions are split between the AP and a Wireless LAN Controller (WLC) — the Split-MAC architecture.

- Lightweight AP handles real-time jobs: transmitting/receiving RF, encrypting/decrypting traffic, and sending beacons and probes.

- WLC handles RF management, security and QoS management, client authentication, and association/roaming management, and centrally configures the lightweight APs.

- The WLC can sit in the same subnet/VLAN as its APs or a different one.

- WLC and APs authenticate each other with X.509 digital certificates installed on each device, so only authorised APs can join.

- They communicate using CAPWAP (Control And Provisioning of Wireless Access Points), based on the older LWAPP (Lightweight Access Point Protocol).

- Control tunnel (UDP 5246) — configures and manages the APs; encrypted by default.

- Data tunnel (UDP 5247) — all wireless-client traffic is tunnelled to the WLC (it does not go straight to the wired network); not encrypted by default, but DTLS (Datagram Transport Layer Security) can be enabled.

- Because client traffic is tunnelled in CAPWAP, lightweight APs connect to switch access ports, not trunk ports.

📌  Benefits of Split-MAC

| Benefit | What it gives you |
| --- | --- |
| Scalability | One WLC (or several) makes building and supporting thousands of APs far simpler. |
| Dynamic channel assignment | The WLC automatically selects each AP's channel. |
| Transmit power optimisation | The WLC sets the appropriate transmit power for each AP. |
| Self-healing coverage | If an AP fails, the WLC raises nearby APs' power to fill the coverage hole. |
| Seamless roaming | Clients roam between APs with no noticeable delay. |
| Client load balancing | A client in range of two APs is associated with the least-used AP. |
| Security / QoS management | Central policy management keeps security and QoS consistent network-wide. |

Lightweight AP modes

| Mode | Behaviour |
| --- | --- |
| Local | Default — the AP offers one or more BSSs for clients to associate with. |
| FlexConnect | Like Local, but can locally switch traffic between wired and wireless if the CAPWAP tunnels to the WLC go down. |
| Sniffer | No client BSS; captures 802.11 frames and sends them to software such as Wireshark. |
| Monitor | No client BSS; receives 802.11 frames to detect rogue devices and can send de-authentication messages to disassociate them. |
| Rogue Detector | Radio off; listens on the wired network and correlates ARP data with a WLC list of suspected rogue MACs to detect rogues. |
| SE-Connect | No client BSS; dedicated RF spectrum analysis on all channels, feeding tools such as Cisco Spectrum Expert. |
| Bridge / Mesh | Dedicated bridge between sites (even long distances); APs can form a mesh. |
| Flex + Bridge | Adds FlexConnect local-forwarding to a bridge/mesh node so traffic still forwards if the WLC link is lost. |

Cloud-based APs (Meraki)

- Cloud-based AP architecture sits between autonomous and Split-MAC: autonomous-style APs that are centrally managed from the cloud. Cisco Meraki is a popular example.

- The Meraki dashboard configures APs, monitors the network, generates reports, and tells each AP its channel and transmit power.

- Only management/control traffic goes to the cloud — client data still goes directly to the wired network, as with autonomous APs.

📌  WLC deployment models (Split-MAC)

| Model | Where the WLC runs | Approx AP scale |
| --- | --- | --- |
| Unified | Hardware appliance in a central location. | ~6000 APs (add more WLCs for more) |
| Cloud-based | A VM on a server, usually a private-cloud data centre (not the same as cloud-based APs). | ~3000 APs (add more VMs) |
| Embedded | Integrated inside a switch. | ~200 APs (add more switches) |
| Mobility Express | Integrated inside an AP. | ~100 APs (add more such APs) |

WLAN configuration on the WLC (GUI)

📋 Exam ref: 2.7 / 2.8 / 2.9 / 5.10 — WLAN components, management access, configure a WLAN (WPA2-PSK) via the WLC GUI

Topology & connections

- APs are usually powered by PoE (Power over Ethernet) — the Ethernet cable carries both power and data.

- The WLC connects to the switch through a LAG (Link Aggregation Group, i.e. EtherChannel). On CCNA-era (AireOS) WLCs the LAG must be static (mode on) — they do not support PAgP or LACP negotiation. (Newer Catalyst 9800 WLCs do support LACP.)

- On first boot you can terminate the setup wizard; alternatively, auto-install can pull the WLC's configuration from a TFTP server.

- AP-to-WLC discovery: DHCP Option 43 can tell APs the WLC's IP address; the WLC then hears the APs' broadcast CAPWAP discovery messages.

WLC ports vs interfaces

- In WLC terminology a port is a physical port, while an interface is a logical construct within the device (like an SVI on a switch).

WLC ports (physical)

| Port | Purpose |
| --- | --- |
| Service Port | Dedicated out-of-band (OOB) management. Supports one VLAN, so it connects to a switch access port. Usable while the WLC boots and for system recovery. |
| Distribution System Port | Standard network ports to the wired distribution system, carrying data traffic. Usually connect to switch trunk ports; multiple such ports can form a LAG. |
| Console Port | Standard console access (RJ-45 or USB). |
| Redundancy Port | Connects a second WLC to form a high-availability (HA) pair. |

WLC interfaces (logical)

| Interface | Purpose |
| --- | --- |
| Management | Management traffic (Telnet, SSH, HTTP, HTTPS, RADIUS, NTP, Syslog). CAPWAP tunnels are formed to/from this interface. |
| Redundancy Management | In an HA pair (two WLCs joined by their redundancy ports, one active / one standby), used to reach and manage the standby WLC. |
| Virtual | Used to communicate with wireless clients — relays DHCP requests and performs client web authentication. |
| Service-Port interface | Bound to the service port and used for out-of-band management. |
| Dynamic | Maps a WLAN to a VLAN (e.g. traffic from the internal WLAN reaches the wired network via its dynamic interface). |

Creating a WLAN — security (GUI)

- In the WLAN's Security tab, set the Layer 2 Security: for CCNA select WPA+WPA2.

- Under Authentication Key Management, select PSK (Pre-Shared Key); set the PSK Format to ASCII.

- Layer 3 Security offers Web Policy options:

- Web Authentication — after the client gets an IP and tries to open a web page, it must enter a username and password.

- Web Passthrough — no credentials; the client just accepts a displayed statement/agreement to gain access.

- Conditional redirect / Splash-page redirect — similar to the above, but additionally require 802.1X Layer 2 authentication.

WLAN QoS profiles

| Profile | Typical use |
| --- | --- |
| Platinum | Voice |
| Gold | Video |
| Silver | Best effort |
| Bronze | Background |

# Part 11 — IP Services

📋 Exam ref: 4.0 — IP Services — 10% of exam

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| DHCP | Dynamic Host Configuration Protocol | Automatically assigns IP addresses and network settings to devices. |
| DNS | Domain Name System | Translates domain names (e.g. google.com) into IP addresses. |
| NAT | Network Address Translation | Remaps private IP addresses to a public IP, allowing multiple devices to share one public IP. |
| PAT | Port Address Translation | Form of NAT mapping multiple private IPs to one public IP using different port numbers. |
| QoS | Quality of Service | Techniques that prioritise traffic to ensure performance for delay-sensitive applications. |
| DSCP | Differentiated Services Code Point | IP header field used by QoS to classify and prioritise different traffic types. |

## DHCP

📋 Exam ref: 4.3 / 4.6 — DHCP and DNS, configure DHCP client and relay

- DHCP — automatically assigns IP address, subnet mask, default gateway, and DNS server to devices.

- DHCP relay (ip helper-address) — forwards DHCP broadcasts to a DHCP server on another subnet.

## DNS

- DNS — resolves domain names to IP addresses (e.g. google.com → 8.8.8.8).

## NAT / PAT

📋 Exam ref: 4.1 — Configure and verify inside source NAT

⚠ GAP — 4.1: NAT (Static and Pool)  (Add notes from video when studied)

Cover: purpose (translate private to public), inside/outside local/global addresses, static NAT (1:1), dynamic NAT (pool), PAT/overload (many-to-one), configuration commands.

## NTP

📋 Exam ref: 4.2 — Configure and verify NTP

⚠ GAP — 4.2: NTP (Network Time Protocol)  (Add notes from video when studied)

Cover: purpose (synchronise clocks), stratum levels, ntp server/ntp master commands, show ntp status, show ntp associations.

## SNMP

📋 Exam ref: 4.4 — SNMP in network operations

⚠ GAP — 4.4: SNMP (Simple Network Management Protocol)  (Add notes from video when studied)

Cover: purpose (monitor/manage devices), manager/agent/MIB, SNMPv2c vs v3, traps vs polling.

## Syslog

📋 Exam ref: 4.5 — Syslog features

⚠ GAP — 4.5: Syslog  (Add notes from video when studied)

Cover: purpose (centralised logging), severity levels 0–7 (Emergency to Debug), logging commands.

## FTP and TFTP

📋 Exam ref: 4.9 — Capabilities and functions of TFTP/FTP

- TFTP (Trivial File Transfer Protocol) — simple, UDP-based, no authentication. Used to transfer IOS images and config files.

- FTP (File Transfer Protocol) — TCP-based, supports authentication. More reliable than TFTP.

# Part 12 — Security Fundamentals

📋 Exam ref: 5.0 — Security Fundamentals — 15% of exam

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| ACL | Access Control List | Rules used to filter network traffic, permitting or denying packets based on IP, port, or protocol. |
| AAA | Authentication, Authorization, Accounting | Security framework controlling access, tracking activity, and logging usage. |
| RADIUS | Remote Authentication Dial-In User Service | Protocol for centralised authentication, authorisation, and accounting. |
| TACACS+ | Terminal Access Controller Access-Control System Plus | Cisco-preferred AAA protocol that separates authentication, authorisation, and accounting. |
| NGFW | Next-Generation Firewall | Advanced firewall with deep packet inspection, application awareness, and IPS. |
| NGIPS | Next-Generation Intrusion Prevention System | Security system that detects and prevents network intrusions in real-time. |

## Key Security Concepts

📋 Exam ref: 5.1 / 5.2 — Security concepts and program elements

- Vulnerability — a weakness in a system that could be exploited.

- Threat — a potential danger that could exploit a vulnerability.

- Exploit — the actual use of a vulnerability to cause damage.

- Mitigation — actions taken to reduce or eliminate a risk (patches, firewalls, training, access control).

## Password Policy

📋 Exam ref: 5.4 — Security password policy elements

- MFA (Multi-Factor Authentication) — something you know (password) + something you have (token) + something you are (biometric).

## ACLs (Access Control Lists)

📋 Exam ref: 5.6 — Configure and verify access control lists

What an ACL is

- An ACL (Access Control List) is a packet filter: it instructs the router to permit or discard specific traffic — the name reflects that it controls access to the network.

- ACLs can match on source/destination IP addresses and source/destination Layer 4 ports (among other fields), and are commonly used from a security perspective.

How ACLs work

- An ACL is an ordered sequence of ACEs (Access Control Entries).

- It is created in global configuration mode, but has no effect until it is applied to an interface, inbound or outbound.

- The router checks a packet against the ACEs from top to bottom. On the first match it takes that action (permit or deny) and stops — every entry below the match is ignored, so the order of entries matters.

- Only one ACL can be applied per interface per direction (one inbound and one outbound).

- Implicit deny — every ACL ends with an invisible “deny any”, so a packet that matches no ACE is dropped.

ACL types

ACL types

| Type | Matches on | Numbered ranges | Named? |
| --- | --- | --- | --- |
| Standard | Source IP address only | 1–99 and 1300–1999 | Yes |
| Extended | Source & destination IP, protocol, source/destination ports | 100–199 and 2000–2699 | Yes |

- Standard ACL placement — apply as close to the destination as possible (they match only source IP, so placing them near the source risks blocking more than intended).

- Extended ACL placement — apply as close to the source as possible, to drop unwanted traffic early.

- Numbered ACLs are identified by a number; named ACLs are configured in a named-ACL config mode, adding one entry per line.

Wildcard masks & entry ordering

- For a single host (a /32 match) you don’t need to specify a wildcard mask — use the host keyword or just the address.

- The router may re-order /32 host entries to make ACL processing more efficient. This changes only efficiency, not the effect of the ACL, and applies to both standard named and numbered ACLs.

Standard ACL commands (IOS)

Standard ACL commands

| Command | Purpose |
| --- | --- |
| access-list <1-99> {permit¦deny} <source> [wildcard] | Add an entry to a standard numbered ACL. e.g. access-list 1 deny host 1.1.1.1 / access-list 1 permit any. |
| access-list <n> remark <text> | Add a description to the ACL (like an interface description). |
| ip access-list standard <name> | Enter standard named-ACL config mode, then add: [seq] {permit¦deny} <source> [wildcard]. |
| ip access-group <acl> {in¦out} | (interface config) Apply the ACL to an interface in the chosen direction. |
| show access-lists / show ip access-lists | Show all ACLs / show only IP ACLs. |

Extended ACLs

- Extended ACLs work mostly like standard ACLs — they can be numbered or named and are processed top to bottom — but they match on more parameters, so they are more precise (and more complex). Numbered ranges are 100–199 and 2000–2699 (see the ACL types table above).

- The main match parameters are: Layer 4 protocol/port, source address and destination address.

- A packet must match ALL of the values specified in an entry. If it matches every parameter except one (e.g. correct source and destination but wrong port), it does not match that entry, and processing continues down the list.

What an extended ACE matches

- Protocol — an IP protocol number, or a protocol name (e.g. tcp, udp, icmp, or ip for all).

- Source / destination IP — for a single host (/32) you must use the host keyword or an explicit wildcard. Unlike standard ACLs, you cannot write the bare address on its own.

- Ports (TCP/UDP) — optionally match the source and/or destination port using an operator (below).

Port operators

| Operator | Meaning | Example |
| --- | --- | --- |
| eq | Equal to | eq 80 → port 80 |
| neq | Not equal to | neq 80 → any port except 80 |
| gt | Greater than | gt 80 → 81 and above |
| lt | Less than | lt 80 → 79 and below |
| range | Inclusive range | range 80 100 → ports 80 to 100 |

Extended ACL command syntax

Extended ACL commands

| Command | Purpose |
| --- | --- |
| access-list <100-199> {permit¦deny} <proto> <src> <src-wc> [op port] <dst> <dst-wc> [op port] | Add an entry to an extended numbered ACL. e.g. access-list 100 permit tcp 10.0.0.0 0.0.255.255 host 2.2.2.2 eq 443. |
| ip access-list extended <name> | Enter extended named-ACL config mode, then add entries (one per line). |
| ip access-list extended <number> | (Modern IOS) configure a numbered ACL with named-style subcommands — but the running-config still displays it in the traditional access-list format. |
| ip access-group <acl> {in¦out} | (interface config) Apply the ACL to an interface in the chosen direction. |

Editing ACLs — sequence numbers & resequencing

- In named/extended ACL config mode every entry has a sequence number, which makes editing much easier:

- Delete a single entry — no <sequence-number> (from within the ACL). This is the big advantage of named-ACL config mode.

- Insert an entry between others — start the line with the sequence number you want it to take.

- Editing a numbered ACL from global config (access-list …) — you cannot delete individual entries; you can only remove the whole ACL with no access-list <number>.

- Resequence — ip access-list resequence <acl-id> <starting-seq> <increment>. e.g. ip access-list resequence 1 10 10 renumbers the first entry to 10 and steps by 10, leaving gaps so you can insert entries later.

- View just the ACLs — do show running-config | section access-list.

💡 Reminder — extended ACE syntax (the mistakes to avoid)

Fixed field order: permit/deny → protocol → source [operator port] → destination [operator port]. Source always comes before destination, and a port operator attaches to the address immediately before it.

Which port slot: a source-port filter goes right after the source; a destination-port filter (the usual case, e.g. a server’s eq 443) goes after the destination.

One host format only: host 2.2.2.2  OR  2.2.2.2 0.0.0.0 — never both.

One protocol only: write tcp, not “ip tcp”.

Example — allow 10.0.0.0/16 to server 2.2.2.2 over HTTPS:  permit tcp 10.0.0.0 0.0.255.255 host 2.2.2.2 eq 443

Example — block source UDP ports 20000–30000 to server 3.3.3.3:  deny udp any range 20000 30000 host 3.3.3.3

## AAA

📋 Exam ref: 5.8 — Authentication, Authorization, and Accounting

- Authentication — verifies who you are.

- Authorisation — determines what you can do.

- Accounting — logs what you did.

- RADIUS — open standard AAA. Combines authentication and authorisation.

- TACACS+ — Cisco proprietary. Separates all three AAA functions. Preferred for device administration.

## Layer 2 Security

📋 Exam ref: 5.7 — Layer 2 security features

⚠ GAP — 5.7: DHCP Snooping, Dynamic ARP Inspection, Port Security  (Add notes from video when studied)

Cover: DHCP snooping (trusted/untrusted ports), Dynamic ARP Inspection (validates ARP), Port Security (limit MACs per port, violation actions: protect/restrict/shutdown).

## VPNs

📋 Exam ref: 5.5 — IPsec remote access and site-to-site VPNs

⚠ GAP — 5.5: IPsec VPNs  (Add notes from video when studied)

Cover: purpose (encrypt traffic over public internet), site-to-site VPN (connects two offices), remote access VPN (connects a user), IPsec phases, IKE.

## Wireless Security

📋 Exam ref: 5.9 / 5.10 — Wireless security protocols, configure WLAN with WPA2

- WEP — Wired Equivalent Privacy. Outdated and insecure. Do not use.

- WPA — Uses TKIP. Improved over WEP but also outdated.

- WPA2 — Current standard. Uses AES-CCMP. Significantly stronger.

- WPA3 — Latest standard. Stronger encryption, brute-force protection. Becoming the new default.

⌨  Key Commands for this section

| Command | Purpose | Description | Device |
| --- | --- | --- | --- |
| access-list <num> <permit/deny> <conditions> | Create an ACL | Configures a standard or extended ACL to filter traffic. | Both |
| ip access-group <ACL-num> <in/out> | Apply ACL to interface | Applies the ACL to an interface for inbound or outbound filtering. | Both |
| aaa new-model | Enable AAA | Enables AAA services for centralised authentication and accounting. | Both |
| radius-server host <IP> | Configure RADIUS server | Specifies the IP address of the RADIUS server for authentication. | Both |
| tacacs-server host <IP> | Configure TACACS+ server | Specifies the IP address of the TACACS+ server for authentication. | Both |
| show access-lists | View configured ACLs | Lists all access control lists currently configured on the device. | Both |
| show authentication | View auth status | Shows the current authentication configuration and status. | Both |
| debug aaa authentication | Debug AAA auth | Monitors authentication processes in real time. | Both |

# Part 13 — Automation & Programmability

📋 Exam ref: 6.7 — Terraform and Ansible — Automation and Programmability

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| IaC | Infrastructure as Code | Managing infrastructure using code/config files instead of manual CLI or GUI. |
| IaC | IaC tool types | Configuration Management (Ansible, Puppet, Chef) vs Provisioning (Terraform). |
| HCL | HashiCorp Configuration Language | Declarative language used to write Terraform configuration files. |
| DSL | Domain-Specific Language | A language built for a specific purpose (e.g. HCL for infrastructure). Less verbose than general languages like Python or Go. |
| YAML | YAML Ain't Markup Language | Human-readable data format. Used for Ansible playbooks and many other config files. |
| VM | Virtual Machine | Software-based computer running on physical hardware. Commonly provisioned by tools like Terraform. |

## Infrastructure as Code (IaC)

- IaC is the practice of provisioning and managing infrastructure (servers, networks, cloud resources) using machine-readable configuration files instead of manual CLI or GUI configuration.

- Benefits: consistency, scalability, repeatability, and speed. The same config file produces the same result every time.

- Two main categories of IaC tools:

  - Configuration Management — Ansible, Puppet, Chef. Manage existing infrastructure: install software, configure settings, maintain state.

  - Infrastructure Provisioning — Terraform. Create, modify, and delete infrastructure resources from scratch (VMs, networks, storage).

Key difference: Configuration management tools work on already-existing systems. Provisioning tools build infrastructure from scratch. They are complementary — Terraform builds it, Ansible configures it.

### Mutable vs Immutable Infrastructure

|  | Mutable | Immutable |
| --- | --- | --- |
| Used by | Configuration management (Ansible, Puppet, Chef) | Provisioning tools (Terraform) |
| How changes work | Modified in place — existing resource is updated | Replace the resource with a new version from scratch |
| Risk | Configuration drift — devices gradually diverge from desired state | No drift — every deployment starts from a clean, predefined state |
| Example | Apply a patch to a running server | Destroy old server, deploy new server with patch baked in |

### Procedural vs Declarative

|  | Procedural (Imperative) | Declarative |
| --- | --- | --- |
| Also called | Imperative | Desired state |
| Focus | HOW — explicit steps in order | WHAT — define the end state, tool works out the steps |
| Example tools | Traditional CLI scripting, some Ansible use cases | Terraform, OSPF network command |
| Pros | More control over exact sequence | Easier to maintain, ensures consistency |

- Procedural example: Configure hostname R1 → assign IP 192.168.1.1/24 to G0/1 → no shutdown. Each step explicit.

- Declarative example: "Router R1, G0/1 = 192.168.1.1/24, interface enabled." The tool figures out what needs to change.

## Ansible

- Developed by Red Hat. Written in Python. Playbooks are written in YAML.

- Configuration management tool — manages existing infrastructure by installing software, configuring settings, and maintaining system state.

- Ensures consistency by applying and enforcing configurations across many devices simultaneously.

- Uses a push model — the control node pushes configs to managed devices.

- Agentless — no software agent required on the managed devices. Uses SSH (Linux) or WinRM (Windows).

### Ansible Key Files

| File | Purpose |
| --- | --- |
| Playbook | YAML file defining tasks to run on managed hosts. The main "script" of Ansible. |
| Inventory | List of managed hosts (IP addresses or hostnames) that Ansible will connect to. |
| Templates | Jinja2 template files used to generate config files with variable substitution. |
| Variables | Files or inline definitions that store values used in playbooks and templates. |

How it works: The control node (your machine running Ansible) reads the playbook, connects to the devices listed in the inventory, and pushes the configuration. No agent needed on the devices.

## Puppet and Chef

- Both are configuration management tools — same category as Ansible.

- Puppet and Chef use a pull model — agents installed on managed devices periodically pull their config from a central server.

- Agent-based — require a software agent installed on each managed device (unlike Ansible which is agentless).

- Use declarative language to define desired system state.

## Terraform

- Open-source IaC provisioning tool developed by HashiCorp (acquired by IBM in 2025).

- Primarily a provisioning tool — creates, modifies, and deletes infrastructure resources (VMs, networks, storage, cloud services).

- Declarative — you define the desired end state in config files; Terraform figures out what to create, change, or destroy.

- Uses a push model and is agentless — like Ansible, no agent required on the infrastructure it manages.

- Supports 100+ providers — AWS, Azure, GCP, Kubernetes, and Cisco platforms including Catalyst Centre, ACI, and IOS XE.

- Configuration files written in HCL (HashiCorp Configuration Language) — a DSL designed specifically for defining infrastructure.

- Core engine written in Go. HCL config files are what you write as the engineer.

- Maintains a state file that tracks all deployed infrastructure. Terraform compares current state vs desired state to determine what changes to make.

### Terraform Workflow (Write → Plan → Apply)

| Step | What happens |
| --- | --- |
| 1. Write | Define the desired infrastructure state in .tf configuration files using HCL. |
| 2. Plan | Terraform compares config files against the current state file and shows a preview of changes. No changes are made yet. |
| 3. Apply | Execute the plan — Terraform provisions, modifies, or destroys resources to reach the desired state. |

Terraform + Ansible together: Terraform provisions the infrastructure (builds the VMs, networks, storage). Ansible then configures and manages those resources (installs software, sets configs). They complement each other.

## Network Automation Fundamentals

📋 Exam ref: 6.1 / 6.2 — Impact of automation on network management; traditional vs controller-based networking

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| ASIC | Application-Specific Integrated Circuit | Purpose-built chip that forwards data-plane traffic far faster than the CPU. |
| TCAM | Ternary Content-Addressable Memory | High-speed memory the ASIC uses to store forwarding tables (e.g. the MAC table). |
| CAM | Content-Addressable Memory | Another name for the MAC address table. |
| SDA | Software-Defined Architecture | Another name for SDN / controller-based networking. |

Why automate

- Traditionally, engineers manage devices one at a time via the CLI over SSH. Downsides: small mistakes and typos are common; it is slow and inefficient at scale; and it is hard to keep every device on the organisation’s standard configuration.

- Network automation benefits — reduces human error; makes the network far more scalable (deployments, network-wide changes and troubleshooting take a fraction of the time); helps assure network-wide policy and compliance (standard configs, software versions); and lowers opex (operating expenses) because each task takes fewer man-hours.

- Tools and approaches include SDN, Ansible, Puppet and Python scripts — one script can configure many devices at once.

Logical planes (data / control / management)

- A device’s functions divide into three logical planes.

- Data plane (forwarding plane) — everything involved in forwarding user traffic between interfaces. A router matches the most specific route and forwards to the next hop, de-encapsulating the old Layer 2 header and re-encapsulating for the next hop’s MAC; a switch forwards by destination MAC (or floods). It also adds/removes VLAN tags, performs NAT, and applies permit/deny decisions from ACLs and port security.

- Control plane — the functions that build the tables the data plane uses (routing table, MAC table, ARP table, STP). It controls what the data plane does: OSPF doesn’t forward packets but tells the data plane how; STP doesn’t forward frames but decides which links are used; ARP isn’t user data but builds the ARP table. Traditionally the control plane is distributed — every device runs its own.

- Management plane — protocols used to manage devices: SSH/Telnet (CLI access), Syslog (logging), SNMP (monitoring) and NTP (time). Like the control plane it is overhead work, but it does not directly affect data-plane forwarding.

CPU vs ASIC (hardware forwarding)

- Control-plane and management-plane work is usually handled by the CPU.

- That is too slow for the data plane, so a purpose-built ASIC handles forwarding. On a switch the MAC table is held in TCAM (a MAC table is also called a CAM table); the ASIC feeds a frame’s destination MAC into the TCAM, gets the matching entry, and forwards it. Modern routers use the same approach (ASIC + TCAM).

- Summary — traffic destined for the device itself (control/management) is processed by the CPU; pass-through data traffic is handled by the ASIC for maximum speed. SDN (next section) builds on these planes by centralising the control plane.

📋 Exam ref: 6.3 — Software-Defined Networking | 6.4 — Cisco SD-Access and DNA Centre

📌  Key Acronyms for this section

| Acronym | Full Name | Description |
| --- | --- | --- |
| SDN | Software-Defined Networking | Approach that centralises the control plane into a controller application, separating it from the data plane on network devices. |
| SBI | Southbound Interface | The interface/API used for communication between the SDN controller and the network devices it controls. |
| NBI | Northbound Interface | The interface/API used for engineers/applications to communicate with and program the SDN controller. |
| IBN | Intent-Based Networking | Approach where an engineer defines the desired network behaviour (intent) and the controller handles implementation details. |
| DNA | Digital Network Architecture | Cisco's framework for intent-based networking. DNA Centre is the controller that implements it. |
| VXLAN | Virtual Extensible LAN | Tunnelling protocol used by SD-Access to build the overlay network. Encapsulates Layer 2 frames in UDP. |
| LISP | Locator ID Separation Protocol | Control plane protocol used in SD-Access. Separates device identity (EID) from location (RLOC). |
| EID | Endpoint Identifier | In LISP/SD-Access: identifies an end host connected to an edge switch. |
| RLOC | Routing Locator | In LISP/SD-Access: identifies the edge switch used to reach a particular end host (EID). |
| IS-IS | Intermediate System to Intermediate System | Link-state routing protocol used in SD-Access Greenfield underlay (instead of OSPF). |
| NETCONF | Network Configuration Protocol | Protocol used over the SBI to configure network devices programmatically. Uses XML. |
| RESTCONF | REST Configuration Protocol | Lighter REST-based version of NETCONF for programmatic device configuration. Uses JSON or XML. |

## Software-Defined Networking (SDN)

- SDN centralises the control plane into an application called a controller — rather than each device running its own distributed control plane.

### Traditional vs SDN Architecture

|  | Traditional Networking | SDN |
| --- | --- | --- |
| Control plane | Distributed — every device has its own control plane | Centralised — controller handles control plane for all devices |
| Routing info | Devices use OSPF/EIGRP to share info with each other | Devices share info with the controller; controller calculates routes |
| Policy (ACLs) | Configured per device — can grow to thousands of entries | Defined centrally as intent; controller pushes configs to devices |
| Config method | SSH/console, one device at a time | Controller GUI or API — applies across entire network |
| Deployment | Slow — manual per-device config before deployment | Fast — new devices auto-receive config from controller |

Key idea: In SDN the control plane (deciding where traffic goes) is separated from the data plane (actually forwarding the traffic). The controller owns the control plane; devices just forward.

### Northbound and Southbound Interfaces

- Southbound Interface (SBI) — communication between the controller and the network devices it controls.

  - Protocols used over the SBI: NETCONF, RESTCONF, as well as traditional protocols like Telnet, SSH and SNMP. Other SBI examples include OpenFlow, Cisco OpFlex and Cisco onePK.

- Northbound Interface (NBI) — allows engineers and applications to interact with and program the controller.

  - DNA Centre exposes a REST API on the NBI — scripts and applications use it to query or configure the network programmatically.

### Cisco SDN Solutions

| Solution | Use case | Controller |
| --- | --- | --- |
| SD-Access | Campus LAN automation | DNA Centre (Digital Network Architecture Centre) |
| ACI | Data centre automation | APIC (Application Policy Infrastructure Controller) |
| SD-WAN | WAN automation | vManage |

## Cisco SD-Access

- SD-Access is Cisco's SDN solution for automating campus LANs.

- DNA Centre is the controller at the centre of SD-Access. It is installed on Cisco UCS server hardware.

- DNA Centre has two roles:

  - SDN Controller — in an SD-Access network.

  - Network Manager — in a traditional (non-SD-Access) network, acting as a centralised management platform.

### Fabric: Underlay, Overlay, and Fabric

| Layer | What it is | Technology used |
| --- | --- | --- |
| Underlay | The physical network — devices and connections providing IP connectivity | IS-IS routing protocol (Greenfield), routed ports, no STP |
| Overlay | The virtual network built on top of the underlay — logical tunnels between edge nodes | VXLAN (Virtual Extensible LAN) tunnels |
| Fabric | The combination of overlay + underlay — the complete SD-Access network | LISP (control plane) + VXLAN (data plane) + CTS (policy) |

Memory hook: Underlay = physical pipes. Overlay = virtual tunnels through those pipes. Fabric = both together, making it work as one network.

### SD-Access Switch Roles

- Edge Nodes — connect to end hosts. Act as the default gateway for end hosts (routed access layer).

- Border Nodes — connect SD-Access to devices outside the domain (e.g. WAN routers, internet).

- Control Nodes — run LISP to perform control plane functions (mapping EIDs to RLOCs).

### LISP — Control Plane of SD-Access

- LISP separates device identity (EID) from location (RLOC).

- EID (Endpoint Identifier) — identifies an end host connected to an edge switch.

- RLOC (Routing Locator) — identifies the edge switch that can reach that end host.

- Control nodes maintain the EID-to-RLOC mapping table so any edge node can find how to reach any end host.

### Greenfield vs Brownfield Deployment

| Type | Description | DNA Centre role |
| --- | --- | --- |
| Greenfield (new deployment) | Fresh network built from scratch. DNA Centre configures the underlay. All switches are Layer 3, IS-IS routing, routed ports, no STP needed. | Configures both underlay and overlay |
| Brownfield (existing network) | SD-Access added on top of existing network hardware/software. DNA Centre does not configure the underlay (left as-is). | Configures overlay only |

### Cisco TrustSec (CTS) — Policy Control

- CTS provides policy control — QoS, security policies, and group-based access control across the fabric.

- Engineers define intent (e.g. "this group of users cannot communicate with this group") and DNA Centre implements the detailed ACLs automatically.

## Intent-Based Networking (IBN)

- Traditional problem: ACLs grow to thousands of entries. Intent is forgotten over time. Per-device config is error-prone.

- IBN approach: the engineer communicates the desired behaviour to DNA Centre. DNA Centre translates intent into device configurations and policies automatically.

- Example: "Sales VLAN cannot access Finance servers." DNA Centre pushes the appropriate ACLs to every relevant device — no manual per-device config.

### DNA Centre Network Management Benefits

|  | Traditional Management | DNA Centre Management |
| --- | --- | --- |
| Configuration | Per-device via SSH or console | Centrally managed via DNA Centre GUI or REST API |
| Policies | Distributed — manually applied per device | Centrally defined and pushed automatically |
| Software updates | Manual per device | DNA Centre monitors cloud for new versions, auto-updates devices |
| New deployments | Slow — manual config required before deployment | Fast — new devices auto-receive config from DNA Centre |
| Error risk | Higher — more manual steps | Lower — automation reduces human error |

⚠ GAP — 6.5: REST-based APIs (CRUD, HTTP verbs) and JSON / data encoding (Add notes from video when studied)

# Exam Topic Coverage Tracker

✅ = confident   🔄 = in progress   ⚠ = not yet studied

| Exam Ref | Topic | Weight | Status |
| --- | --- | --- | --- |
| 1.1 | Network components (routers, switches, firewalls, APs, PoE) | 20% | 🔄 |
| 1.2 | Network topology architectures (2-tier, 3-tier, spine-leaf) | 20% | 🔄 |
| 1.3–1.4 | Cabling, interface types, cable issues | 20% | ✅ |
| 1.5 | TCP vs UDP | 20% | 🔄 |
| 1.6 | IPv4 addressing and subnetting | 20% | ✅ |
| 1.7 | Private IPv4 addressing | 20% | 🔄 |
| 1.8–1.9 | IPv6 addressing and address types | 20% | ⚠ |
| 1.10 | Verify IP parameters (Windows/Mac/Linux) | 20% | ⚠ |
| 1.11 | Wireless principles | 20% | ✅ |
| 1.12 | Virtualisation fundamentals (VMs, containers, VRF) | 20% | ⚠ |
| 1.13 | Switching concepts (MAC learning, flooding, frame switching) | 20% | ✅ |
| 2.1 | VLANs — access ports, default VLAN, inter-VLAN | 20% | 🔄 |
| 2.2 | Interswitch connectivity — trunks, 802.1q, native VLAN | 20% | 🔄 |
| 2.3 | CDP and LLDP | 20% | 🔄 |
| 2.4 | EtherChannel (LACP) | 20% | ✅ |
| 2.5 | Rapid PVST+ Spanning Tree Protocol | 20% | 🔄 |
| 2.6–2.9 | Wireless architectures, WLAN configuration | 20% | ✅ |
| 3.1–3.2 | Routing table interpretation and forwarding decisions | 25% | ✅ |
| 3.3 | Static routing (default, network, host, floating) | 25% | 🔄 |
| 3.4 | Single area OSPFv2 | 25% | ⚠ |
| 3.5 | First Hop Redundancy (HSRP/VRRP/GLBP) | 25% | ⚠ |
| 4.1 | NAT (static and pool) | 10% | ⚠ |
| 4.2 | NTP | 10% | ⚠ |
| 4.3/4.6 | DHCP and DNS | 10% | 🔄 |
| 4.4 | SNMP | 10% | ⚠ |
| 4.5 | Syslog | 10% | ⚠ |
| 4.7 | QoS (classification, marking, queuing, policing, shaping) | 10% | 🔄 |
| 4.8 | SSH remote access | 10% | 🔄 |
| 4.9 | FTP / TFTP | 10% | 🔄 |
| 5.1–5.2 | Security concepts and program elements | 15% | 🔄 |
| 5.3–5.4 | Device access control and password policy | 15% | ✅ |
| 5.5 | IPsec VPNs | 15% | ⚠ |
| 5.6 | ACLs | 15% | ✅ |
| 5.7 | Layer 2 security (DHCP snooping, DAI, port security) | 15% | ⚠ |
| 5.8 | AAA concepts | 15% | ✅ |
| 5.9–5.10 | Wireless security (WPA/WPA2/WPA3) | 15% | ✅ |
| 6.1–6.7 | Automation and Programmability | 10% | 🔄 |